ABSTRACT

The report 308 pages, 1book, 14figures,5tables,138 references, 13 appendices.

PLANTING OF VEGETABLES, FLEXIBLE DRIP IRRIGATION TAPES, MICROCLIMATIC CONDITIONS, MULCHING FILM, REMOVAL OF POLYMER RESIDUES, HYDRAULIC DRIVE

The object of the research is technology and means for removing polymer residues from the field surface in the post-harvest period.

The purpose of the work is to increase the efficiency of soil mulching and drip irrigation by developing mechanized means of removing them from the field surface in the post-harvest period.

In the process of work, theoretical and experimental studies were carried out to substantiate the parameters and operating modes of the mechanisms of the device for removing mulch film and flexible irrigation tapes for drip irrigation in the post-harvest period from the field surface.

As a result of the research, a technology and means for its implementation were developed for joint and separate harvesting of mulch film and flexible drip irrigation tapes in the post-harvest period.

The main design and technical and operational indicators: versatility of the developed device for joint and separate cleaning of polymer residues; low cost of equipment.

The degree of implementation - testing of the developed technology and technical means in economic conditions when harvesting polymer residues has shown practical applicability for large areas of a field with mulch.

The efficiency of the unit is estimated by reducing the labor intensity of the process by introducing mechanized technology to intensify the production of vegetables.

| CONTENT |  |  |
| --- | --- | --- |
|  | INTRODUCTION …………………………………………………… | 8 |
| 1 | STATE OF VEGETABLE GROWING WITH THE USE OF MULCH FILM AND DROP IRRIGATION ……………………....... | 11 |
| 1.1 | Necessity of vegetables production and fruits for human nutrition ........…………………………………………………. | 11 |
| 1.2 | Production of fruit and vegetable products in the Republic of Kazakhstan ……………………….......................... | 11 |
| 1.3 | Using plastic film to mulch the soil when growing vegetables………. | 12 |
| 1.4 | Using flexible irrigation belts for drip irrigation of vegetables…………………………………...................................... | 13 |
| 1.5 | Technology and technical means for removing polymer residues after harvesting ………………............................................... | 16 |
| 1.6 | Analysis of the studies studied, setting the goal and objectives of the study to remove polymer residues from the field surface…………………………………………………. | 20 |
| 2 | JUSTIFICATION OF PARAMETERS AND OPERATING MODES OF THE DEVICE FOR REMOVING MULCHING FILM AND FLEXIBLE IRRIGATION TAPES …………………….. | 23 |
| 2.1 | Features of cleaning mulch film and flexible irrigation tapes on the proposed device …............................................................ | 23 |
| 2.2 | Calculation of the pulling forces of the unit for winding mulch film…………………………………………………………….. | 24 |
| 2.3 | Determination of the tensile force of the mulch film during winding on a drum …………………………………………………… | 27 |
| 2.4 | Calculation of the hydraulic drive mechanisms for winding mulch film and drip irrigation tape…………………………. | 28 |
| 3 | THE EXPERIMENTAL STUDIES OF THE INFLUENCE OF FACTORS ON THE TECHNOLOGY OF MOVING OFF OF MULCH FILM AND FLEXIBLE IRRIGATION TAPE....... | 32 |
| 3.1 | Work program ……..…………………………………………………. | 32 |
| 3.2 | Research methodology …….…………………………………………. | 32 |
| 3.3 | Research results ……………………………………………………… | 34 |
| 4 | DEVELOPMENT OF AN EXPERIMENTAL MODEL OF A DEVICE FOR CLEANING OF POLYMER RESIDUES, FIELD TEST AND ESTIMATION OF ECONOMIC EFFICIENCY ……… | 36 |
| 4.1 | Program of experimental work……………………………………….. | 36 |
| 4.2 | Methodology for performing device development, field testing and cost-benefit of the economic efficiency …………………. | 36 |
| 4.2.1 | Conducting patent information research………………………...……. | 36 |
| 4.2.2 | Completing up technical specifications for the development of design documentation ………………………................................... | 36 |
| 4.2.3 | Development of design documentation for the manufacture of an experimental model of a device for removing film from the surface field…………………………………………………. | 36 |
| 4.2.4 | Production of experimental sample device……………………….. …. | 37 |
| 4.2.5 | Field test of the developed device under farm conditions …..……….. | 37 |
| 4.2.6 | Assessment of economic efficiency of development …………………. | 39 |
| 4.3 | Results of work ………………………….. …………………………. | 39 |
| 4.3.1 | Patent-information search …………………………….. …………….. | 39 |
| 4.3.2 | Terms of reference for the development of design documentation for a device for removing polymeric materials from the surface field ……………………………………… | 40 |
| 4.3.3 | Design documentation of a device for removing polymeric materials from the field surface ……………………………………... | 40 |
| 4.3.4 | Experimental device for cleaning mulch film and flexible drip irrigation tapes from the field surface ............................... | 40 |
| 4.3.5 | Results of field tests of the developed device under farm conditions……………………………….............................................. | 40 |
| 4.3.6 | Assessment of economic efficiency of development…………………. | 41 |
|  | CONCLUSION .……………………………………………………… | 42 |
|  | LIST OF REFERENCES....................................................................... | 44 |
|  | APPENDIX A - Technical specification and work schedule ……..…. | 52 |
|  | APPENDIX B - Report of patent research …………………………... | 62 |
|  | APPENDIX W – List of patents and aplications…...………..……….. | 84 |
|  | APPENDIX G - Results of calculations of the theoretical part……… | 92 |
|  | APPENDIX D - Data of experimental study………………………… | 94 |
|  | APPENDIX E - Terms of reference for the development…………… | 104 |
|  | PPENDIX Zh - Design documentation of an experimental model…… | 110 |
|  | APPENDIX I – General view of an experimental model of a machine for removing mulch film and flexible drip tape………... | 111 |
|  | APPENDIX K - Energy performance of machine for remove mulch film and flexible drip tape …………………………… | 112 |
|  | APPENDIX L – Economic efficiency of work……………………… | 136 |
|  | APPENDIX M – Copy of articles…………..……………………… | 148 |
|  | APPENDIX N – Equipment Certificates …..………………………… | 301 |
|  | APPENDIX P – List of articles published in 2018 -2020……………. | 307 |

DESIGNATION

In this report on scientific research work, the following terms are used with appropriate definitions:

| Weather resistance | – | it is the ability of polymeric materials to withstand the effects of various atmospheric factors (solar radiation, heat, atmospheric oxygen, moisture, industrial gases, etc.) for a long time without significant changes in appearance, as well as operational properties (physical and mechanical, dielectric, etc.) |
| --- | --- | --- |
| Biological damage (biological damage) | – | any change (violation) of the structural and functional characteristics of an object caused by a biological factor |
| Soil mulching | – | covering the soil surface with a waterproof material to conserve moisture and enhance plant development |
| Polyethylene | – | synthetic solid material obtained from the polymerization of ethylene |
| Drip irrigation | – | irrigation method in which water is supplied directly to the root zone of the grown plants in controlled small portions using dropper dispensers |
| Flexible irrigation tape | – | flexible polymer tubes with calibrated holes |
| Mulch film harvester | – | device for removing mulch film in the post-harvest period |
| Natural and climatic conditions | – | long-term statistical weather regime typical for a given area due to its geographic location |
| Vegetable crops | – | agricultural vegetable plants grown by humans for the purpose of obtaining edible juicy productive organs, combined under the culinary concept of vegetables |
| Decomposition | – | destruction, decay of a complex object |
| Technological process | – | a set of scientifically and technically grounded methods of influence on changing or defining labor on a material until the required result is obtained |

SYMBOLS AND ABBREVIATIONS

In this report on scientific research work, the following symbols and abbreviations are used with appropriate definitions:

| AIC | – | Agro-industrial complex |
| --- | --- | --- |
| PTO | – | Power take off |
| USA | – | United State of America |
| A0 | – | Initial cross-section of the sample, mm |
| D0 | – | Factory roll diameter, mm |
| D1 | – | Diameter of used film rewound from the field, mm |
| Fmax | – | Maximum tensile load during tensile test, N |
| Fr | – | Tensile load at the moment of rupture, N |
| Fмк | – | Resistance force of the plant stems cutting mechanism, N |
| Fc и Fд | – | Friction force of the vegetable product at the initial moment of box shift and at steady state motion, N |
| Fок | – | Support wheel resistance force, N |
| fc и fд | – | Static and dynamic coefficients of friction |
| Gм | – | Device weight force, N |
| n | – | Number of soil removing discs |
| Рс и Рд | – | Normal pressure force, N |
| Rдм | – | Resistance force of the film edge cleaning mechanism, N |
| R | – | Disc radius, m |
| S | – | Active disk area, m2 |
| α | – | Segment arc angle, degree |
| μкач | – | Rolling friction coefficient |
| σz | – | Tensile strength, MPa |
| σr | – | Breaking strength, MPa |

INTRODUCTION

The project on the theme: "Technology and means of removing mulch film and flexible irrigation tapes of drip irrigation from the field after the harvesting period" was carried out in accordance with the implementation plan of the budget program 217 "Grant funding for scientific research" (Appendix A).

Environmental problems associated with the production and use of polymers are, on the one hand, with their indispensability (due to the uniqueness of their physicochemical and consumer properties), and on the other hand, with the huge volumes of their production and products from them, which, ultimately, they pass into the category of waste hazardous to the natural environment. In particular, a large amount of polyethylene is processed into film materials, which one has to meet in production and everyday life at every step. These are packaging materials - bags and bags for food, various containers for storing various liquids (from water to mineral oils), which, having fulfilled their functions, are thrown away and disrupt the natural cycle of substances in nature. The problem is aggravated by the fact that industrial synthetic polymers such as polyethylene, polypropylene, polystyrene, polyamides, polyesters, etc. are very stable chemical compounds under normal conditions. Many of them are able to withstand the effects of solar radiation and atmospheric oxygen in combination with the effects of heat and moisture in natural conditions for tens of years without noticeable chemical destruction. Fragments are formed from polymers that undergo destruction, which remain in the environment for quite a long time and pollute it for many years [1,2].

Relevance. Ensuring food security of the country, improving the quality of life of the population, are the main tasks facing the agro-industrial complex of Kazakhstan, the solution of which can be carried out only in conditions of sustainable development of the domestic agricultural sector. One of the main directions for solving this problem is the development of plant growing, including vegetable growing to provide food for the population of the republic [3, 4].

According to statistics, the production and consumption of fruits and vegetables per person in the republic in recent years has increased significantly due to a decrease in exports and a slight increase in imports. However, the production of fruits and berries in the country is still two times lower than the consumption rate. The actual consumption of these in different years varied depending on the ratio of exports and imports, but was significantly lower than the established consumption standards [5, 6].

One of the effective methods of increasing the production of vegetables and melons is mulching (from the English word mulch - covering with straw or manure) of the soil. Mulching is covering the surface of the soil with any material. This agricultural technique is used in various climatic zones, in northern countries they mulch the soil around plants, warming them, and in southern countries they save them from heat and strong evaporation of moisture from soil [7, 8, 9].

In the republics of the post-Soviet space, mulching of soil with film on an industrial scale was not successful, due to the lack of polyethylene film, effective technological schemes for their use and technical means for implementing the processes of laying, planting and caring for the crop in conditions of film technology. However, field trials were carried out to identify the effectiveness of mulching, the analysis of which shows that this method is cost effective and does not degrade product quality [10-13].

In recent years, research has been carried out on the use of various materials for mulching the soil using a compressed mass, which includes mineral fertilizers, allopathically active substances of plant origin [14]. Such a composition after harvesting does not require the assembly of mulching material, since they completely turn into humus until next year. However, the cost of such a mass is still very high and unprofitable on large sites. Another direction of intensification of agricultural production of vegetables is to increase the efficiency of irrigation water use. One of the promising ways to solve this problem is to use the method of irrigation - drip irrigation [15]. In Kazakhstan, sown areas with the use of drip irrigation make up only 0.5% of all areas under vegetable and fruit crops. At the same time, in such arid regions as Almaty, Dzhambul and Turkestan (South Kazakhstan), this figure is, respectively, 0.1%, 0.3% and 1.6% [16].

The considered developments on the issue of combining soil mulching with polyethylene tape and drip irrigation are devoted to the work of a number of domestic and foreign researchers [17-26].  Considering the advantages of mulching and drip irrigation, technologies are used to combine them, by laying a flexible irrigation tape under the mulching film by combining film laying and flexible irrigation tapes with a bed former, they are able in one pass to form a ridge with a width of 45 to 90 cm along the upper part using side plowshares and a press roller and cover the soil with foil or covering material with different widths [27-29]. At the same time, the flexible drip irrigation tape fits very easily under the mulch [30-34].

In the post-harvest period of harvest, the plastic film used as mulch remains under the foliage and tops of the plants, which in turn makes it difficult to remove the plastic film from the field. Therefore, the fields are clogged with polymer residues, which in turn make it difficult to cultivate the soil, remaining for years if it gets into the soil. According to the timing data of the authors-applicants of the project, the laboriousness of removing the polyethylene film used as mulch is about 64 people * hour / ha [35].

All of the above, as well as the experience of world practice, practically requires the use of advanced technology for mulching the soil with plastic wrap and laying flexible tapes for drip irrigation in the production of vegetable crops in the republic, taking into account the soil and climatic conditions. This predetermines the need to develop a complex of machines for intensifying the production of vegetables and other crops.

For mulching the soil with plastic wrap and laying flexible irrigation tapes in sufficient quantities, there are developments and serial machines are also produced. However, the equipment for removing mulch film and flexible irrigation belts are lagging behind in quality and are very high in cost, and therefore they are not available to rural producers in Kazakhstan.

In this regard, the development of technology and technical means for removing mulch film and flexible drip irrigation tapes is relevant for the effective use of polymeric materials in agriculture by eliminating environmental hazards for the human environment.

The subject of research is the regularities of technological processes in the mechanized technology of removing polymer residues from the soil surface after the harvesting period.

The goal of the project was to increase the efficiency of soil mulching and drip irrigation by developing mechanized means of removing them in the post-harvest period from the surface of the field to exclude side effects in the environment.

The achievement of the goal provided for the solution of the following important tasks: to develop a technological process for removing used mulching film and flexible irrigation tape in the post-harvest period from the field; theoretically and experimentally substantiate the operating modes of removing mulch film, flexible irrigation tapes and design parameters of the device for its implementation; to conduct laboratory studies of the proposed technology for removing mulching film and flexible irrigation tape during joint and separate work; to test the developed means for mechanized removal of polymer residues from the field surface; to evaluate the technical and economic efficiency of using the technology for removing mulch film and flexible irrigation tape.

The scientific novelty of the project lies: in the development of technology and technical means for removing residues of polymer materials such as mulch and flexible irrigation belts in the post-harvest period; theoretical and experimental substantiation of the parameters of the operating modes of the device for removing mulch film and drip irrigation tapes.

The novelty of the developed technological and technical solutions for the period of the project (2018-2020) is confirmed by patents: Device for removing mulch film (Patent for utility model RK 1782. 12.11.2015. Publ. 15.11.2016, bul. No. 15); A device for determining the friction coefficient of materials (Patent for utility model 3368. Appl. 03.05.2018. Publ. 30.11.2018, bul. No. 45); A device for removing and winding polyethylene films from the surface of the field (Patent for invention RK 34443. Appl. 09/13/2018. Publ. 07/03/2020, bul. No. 26).

All measuring instruments used in scientific research passed in accordance with the terms of the university's measuring instruments calibration schedule. Most of the devices and equipment were purchased in a new form, which had the appropriate certificates with the ability to use in the Eurozone.

As a result of the project implementation by stage, the following interim scientific reports were submitted:

- Technology and means of removing mulch film and flexible irrigation tapes for drip irrigation from the field after the harvesting period, state registration number 0118RK00442, Inv.# 0218RK 00411 (for 2018);

- Technology and means of removing mulch film and flexible irrigation tapes for drip irrigation from the field after the harvesting period, state registration number 0118RK00442, Inv. # 0219RK00814 (for 2019).

1 State of vegetable growing with the use of mulch film and drop irrigation

1.1 Necessity of vegetables production and fruits for human nutrition

The importance of vegetables, fruits and berries in human nutrition is well known. They are delicious, nutritious for healthy. Vegetables and fruits do not have a high energy value, but they are suppliers of essential nutrients (nutrients necessary for the normal implementation of metabolism), on which human health directly depends. Vitamins, minerals, easily digestible carbohydrates, dietary fiber - this is not a complete list of useful substances that are rich in these gifts of nature [36, 37].

For the normal functioning of the gastrointestinal tract, the body needs fiber, which is found in large quantities in vegetables and fruits.

Also, fruits and vegetables remove cholesterol from the human body and normalize fat metabolism. Vegetables and fruits are very well used in the treatment of ailments, increase immunity, as they, in turn, have medicinal properties. The use of fruits and vegetables for medicinal purposes is very important [38].

The components of fruits and vegetables do not harm human health, unlike products made on a synthetic basis. Researches have proven that eating fruits and vegetables every day in your diet can prolong human life by more than 10 to 12 years. This can also be proved by the use of fruits and vegetables in the treatment of various diseases using traditional medicine, in which many remedies are obtained with the use of fruits and vegetables [38].

The chemical composition of vegetable raw materials contains proteins, fats, carbohydrates and lipids, and vegetable raw materials have a capillary-porous structure. Biologically active substances such as vitamins, polyphenols, organic acids and minerals are contained in small quantities, they are responsible for taste and biological value. Vegetables and fruits are also an important source of mineral elements involved in the most important metabolic processes in the human body. The most valuable components of vegetables are various vitamins contained in them in large quantities, which are vital for human life [36, 37, 38, 39].

Lack of any of the vitamins in the human body can lead to serious disorders of various systems and cause serious illness. Vitamins are organic compounds needed to maintain normal metabolism. In order to maintain a healthy lifestyle, a person in the modern world needs to focus on products of plant origin, i.e. increase the diet of vegetables, fruits and berries.

1.2 Production of fruit and vegetable products in the Republic of Kazakhstan

According to statistics, the production and consumption of fruits and vegetables per person in the republic in recent years has increased significantly due to a decrease in exports and a slight increase in imports. However, the production of fruits and berries in the country is still two times lower than the consumption rates per person. The actual consumption of these crops in different years varied depending on the ratio of exports and imports, but was significantly lower than the established consumption standards [40, 41].

At the end of the year, about 90% of the domestic production of the considered vegetables is produced by households and peasant or farms [42]. The rest of the volume falls on agricultural enterprises. This shows once again that there is a problem of small-scale commodity production in Kazakhstan. Domestic production of these vegetables is primarily provided by ordinary agricultural producers. The main producers are 3 regions: Almaty (26.9%), Turkestan (South Kazakhstan) (23.4%) and Dzhambul (17.8%), their total share is 68% of the total gross harvest of the republic. Because on the territory of these three main fruit and vegetable regions, and these are Almaty, Dzhambul and Turkestan (SKO), most of the population of Kazakhstan lives. So, during this period there was an increase in prices for all types of vegetables. The main reasons for the increase in prices for vegetable products were:

- increase in prices for fuels and lubricants;

- increasing the income of the population;

- rise in the cost of services for the transportation of goods;

- inflation rate.

The production and sale of vegetables in Kazakhstan has a pronounced seasonal character, very few early vegetables are grown, therefore, they are import dependent. Hence the inflated prices during the period of decreasing offers. Therefore, the following conclusion should be drawn:

In the Republic of Kazakhstan, the production of vegetable products does not fully meet the domestic demand, despite the fact that the gross harvest and yield are growing every year. Kazakhstan retains import dependence on most vegetable crops;

Kazakhstan has stable domestic demand and imports most of its vegetable crops from neighboring countries (Uzbekistan, China and Kyrgyzstan). Export opportunities in large volumes are limited for the following main reasons:

- non-provision of domestic demand with products of its own production;

- sharply continental climatic conditions that do not allow for year-round cultivation of vegetable products;

- a limited amount of irrigated land suitable for growing vegetables;

- undeveloped technological system for growing vegetable crops;

- lack of a developed infrastructure for processing vegetable crops

- deficiency of irrigated water supply in regions with normal weather conditions.

One of the modern intensive systems for growing vegetables is the use of polymeric materials, in particular for soil mulching and drip irrigation. These systems can be used together or separately, depending on the type of products grown and technology. Despite the significant costs to use, these systems generate huge revenues.

1.3 Using plastic film to mulch the soil when growing vegetables

Mulching the soil with plastic film in the field is one of the few agricultural methods of influencing the complex of factors on which the growth and development of plants depend.

Polyethylene film, as a material for soil mulching, is used in the field and in greenhouses in order to increase the yield and quality of the most valuable food crops [43, 44].

In foreign countries, the largest areas under film mulching are occupied by strawberries. It is also used when growing potatoes for early production and many vegetable crops (cucumbers, tomatoes, watermelons, zucchini). Film mulching is used in fruit plantations and floriculture [44].

In the republics of the former Soviet Union, on an industrial scale, soil mulching with a film was not carried out, since there were not enough plastic films, the most effective technological schemes for their use were not developed, and there were no technical means for implementing the processes of laying, sowing and caring for the crop in conditions film technology [44-47]. However, a number of field testing were undertaken to determine the effectiveness of mulching, analysis of the data, which shows that this method of cultivation is cost effective, does not degrade the quality of the product [48-52].

According to the authors, an increase in crop yields when mulching the soil with a film occurs due to better moisture conservation in the upper soil horizons, an improvement in the thermal regime of the near-soil layer of air and the upper soil layer, weak weediness of crops, saving of the upper soil horizons in a looser state [53, 54].

The use of polyethylene films also avoids the use of herbicides and provides early planting [55].

In addition to transparent film, the industry also produces color film and black film [56]. The data on its effectiveness are contradictory - some researchers point to the lack of effect, others received significant increases in yield. So, in foreign practice, mulching the soil with black plastic film [57] accelerated plant growth, increased early tomato yields by 40%, reduced fruit cracking and improved their quality.

Due to the different planting and seeding schemes of crops when mulching the soil, the width of the plastic film can vary within different limits. Most often, when using films on large areas abroad, the width is up to 140 cm [58, 59]. Most authors note that mulching with a film in wide strips (1m) is more effective than in narrow stripes [60-65].

In recent years, research has been carried out on the use of various materials for mulching the soil using a compressed mass, which includes mineral fertilizers, allopathically active substances of plant origin [66].

Such a composition after harvesting does not require the assembly of mulching material, since they completely turn into humus until next year. However, the cost of such a mass is still very high and unprofitable on large areas.

1.4 Using flexible irrigation belts for drip irrigation of vegetables

Another direction of the intensification of agricultural production, to a greater extent in vegetable growing, is to increase the efficiency of the use of irrigation water and its economy. One of the promising ways to solve this problem is to use the method of drip irrigation.

The study of world experience and the results of many years of experiments with drip irrigation systems in Kazakhstan in various climatic and soil conditions shows that the creation of irrigation systems of this type is effective and economically profitable when irrigating perennial plantations, vegetable, row crops and industrial crops, ornamental plantations, nurseries, etc. It is very effective to use drip irrigation systems with intensive technologies for growing agricultural crops, primarily vegetables, when the size and quality of the resulting crop largely depend on the accuracy of maintaining the moisture regime and diet regime. Consideration that drip irrigation is more expensive than conventional irrigation methods, it must be used with maximum efficiency, and only where it cannot be done without it. These are, first of all, lands with a lack of irrigation water in areas with prolonged droughts [67].

In accordance with the natural conditions in Kazakhstan, the Ministry of Agriculture of the Republic Kazakhstan has allocated two zones for vegetable growing and melon growing - North and South. The southern zone includes Almaty, Dzhambul, Turkestan (South Kazakhstan) and Kyzylorda, other regions of the republic are included in the northern zone.

The main climatic features of these zones, which determine different directions of vegetable growing and melon growing, are characterized by the following indicators. The average duration of a period with temperatures above 10 ° C, when normal growth and development of vegetable crops is possible, in the northern zone is 110-140 days, in the southern - 150-190 days.

The sum of active temperatures is respectively 1900-2000°C and 3200-4000°C. The number of hours of sunshine for the northern regions is 1800-2200, for the southern regions - 2500-2800. These conditions limit the area of growing light-loving vegetable crops in certain western regions of the republic.

In accordance with natural and climatic conditions, the main sown areas for open ground vegetables are located in the southern region of the republic: Turkestan (South Kazakhstan) region occupies 24%, Almaty - 22.5% and Dzhambul - 18.2%.

However, at present in Kazakhstan, as follows from Table 1.1, sown areas with the use of drip irrigation make up only 0.5% of all areas for vegetable and fruit crops. At the same time, in such arid regions as Almaty, Dzhambul and Turkestan (South Kazakhstan) regions, this indicator is 0.1%, 0.3% and 1.6%, respectively [68].

The countries that were the first to introduce drip irrigation: England - 1948, Israel - the mid-50s of the last century, USA - 1969, Australia, France - 70 years, achieved high harvest rates of various crops. The limited water resources suitable for irrigation require the introduction of such irrigation methods that will significantly reduce the consumption of irrigation water. Using of traditional methods of surface and rain irrigation gives at least 30% of moisture loss due to evaporation from the soil surface and transpiration by weeds in the aisles. In addition, these methods contribute to the development of water erosion and soil salinization. The main advantage of drip irrigation is the continuous maintenance of optimum moisture levels in the root layer of the soil. Abroad, in the first years of using drip irrigation, it was noted that the largest increase in drip irrigation is observed in fruit, citrus and vegetable crops by at least 30% [69].

Table 1.1 - Cultivated areas using drip irrigation in the context of regions in the Republic of Kazakhstan

| #/# | Region | Drip irrigation area, ha |
| --- | --- | --- |
| 1 | Atyrau | 217,3 |
| 2 | Aktobe | 88 |
| 3 | Almaty | 47 |
| 4 | Zhambyl | 142 |
| 5 | West Kazakhstan | 140,1 |
| 6 | Kyzylordinskaya | 0,07 |
| 7 | Mangystau | 50,5 |
| 8 | North Kazakhstan | 50 |
| 9 | Turkestan (South Kazakhstan) | 810 |
| Total |  | 1545 |

Another major advantage of drip irrigation is a significant saving of irrigation water, which moistens only the root layer of the soil, excluding deep soaking and evaporation, as well as surface runoff and wind drift. Almost all water is used for transpiration by the plant, that is, for breathing. In foreign systems, the rate of irrigation with water decreased by 20-50%, and in some cases 75-90% compared to the method of watering by flooding. The degree of irrigation water saving is determined by climatic conditions, the drier the area, the higher the efficiency of drip irrigation. In the territories of the former Soviet Union, positive results were also obtained on the use of drip irrigation [70].

It is known that domestically produced drip irrigation systems were introduced in Moldova and the Crimea in the 80s of the last century.  Basically, these projects have failed due to imperfect drippers, poorly resistant to clogging and lack of reliable water treatment systems for irrigation. Israel is a pioneer in drip irrigation and has been continuously working to improve drip and water filtration systems for 40 years. Currently, 4th generation droppers are used, which are highly resistant to clogging [71].

About 70% of the water produced by man is used for the production of agricultural products in the world. Therefore, one of the directions for the intensification of agricultural production in the region is to increase the efficiency of the use of irrigation water. A promising way to solve this problem is the use of drip irrigation [72].

In the peasant farms of the Republic of Kazakhstan engaged in the cultivation of vegetable crops, the volume of use of drip irrigation is slow due to the remoteness from the central power supply, the need for energy costs, the cost of which is constantly growing, and the lack of reliability and efficiency of individual elements of the drip irrigation system.

The introduction of drip irrigation technology on the lands of the Southern region, the most favorable for growing agricultural products in terms of climatic conditions and acutely experiencing a shortage of irrigation water, will allow:

- to significantly reduce the volume of irrigation water consumed;

- to increase the volume of production of vegetables and fruits and berries.

Favorable soil and climatic conditions of the Southern region, the availability of irrigated lands allow for vegetable growing, horticulture, viticulture. The harvest from which can provide not only the needs of the population of the southern region, but also the northern regions of the country.

In modern agricultural production, there are many types of watering or irrigation. They differ from each other in the principle of application, depending on the grown plants, geography of use, cost, water and energy costs, etc. Mainly distinguish: furrow irrigation; by holes (watering by flooding); sprinkling (including micro spray); hose irrigation; drip irrigation, etc. [73].

Taking into account the advantages of mulching and drip irrigation, technologies are used to combine them, by laying a flexible irrigation tape under the mulching film. In the post-harvest period of harvesting, the plastic film used as mulch remains under the foliage and tops of plants, which in turn makes it difficult to remove the plastic film from the field [74].

1.5 Technology and technical means for removing polymer residues after harvesting

As a result of a literature review and a patent search on methods and technical means for removing mulch film and flexible irrigation tapes, the following types of technology for removing polymer residues were established (Figure 1.1). Discovered many designs of machines for removing protective film and tape from the soil [75-89] made it possible to analyze. A brief description of them is presented in Appendix B.

Depending on the conditions of laying, mulching films and flexible irrigation belts can be operated together or separately. The laying condition depends from irrigation technology. Therefore, removal can be carried out in three cleaning methods.

One-phase cleaning, where all operations are performed simultaneously, and it is the most complex and, accordingly, expensive, because multifunctional equipment is used. One-phase cleaning can be done in two ways:

-performing all mechanized operations simultaneously;

-burning plant residues and polymeric materials in the field.

Two-phase harvesting can be done in two ways:

-removal of the mulching film in the first phase, and in the second phase, the drip tape is removed, in this phase, the stalks are trimmed by the combine harvester during harvesting;

-the second method includes phases of cutting the stems and phases of simultaneous removal of mulch and drip tapes.

Figure 1.1 - Classification of methods for removing polymer residues from the surface of the sowing field

Three-phase removal is carried out separately, first pruning the stems, then removing the mulch film separately, and finally removing the drip tape. A review and analysis of methods for removing mulch film and flexible irrigation belts show that various types of technical means are used when performing. These equipment have certain advantages and disadvantages when performing established operations. These indicators are systematized (tables 3.2, 3.3, 3.4).

Table 3.2 - Methods for removing drip tape

| Ways to remove drip tape |  | Advantage | Drawbacks |
| --- | --- | --- | --- |
| Not mechanized (hand-held) |  | - does not require mechanized and other means | - poor efficiency - high labor intensity - high time consuming - not convenient loading - high transportation costs |
| Mechanized |  | - reduction of labor intensity - saving time - ease of loading - incomplete use of the carrying capacity of the vehicle - the complexity of removing the rounded tape | - requires special equipment - does not remove mulch film at the same time - requires an operator to operate |

Table 3.3 - Methods for removing mulch film

| Ways to remove plastic film |  |  | Advantage | Drawbacks |
| --- | --- | --- | --- | --- |
| Without mechanization (hand-held) |  |  | - does not require mechanized and other means | - poor efficiency - high labor intensity - high time consuming - not convenient loading - high transportation costs |
| Flame |  |  | - high performance - does not require loading and transportation | - requires special equipment - environmental pollution - incomplete burning of the mulch film |
| Partially mechanized |  |  | - improves working conditions for working personnel, which increases operation rate | - requires special equipment - high labor intensity of the process - time consuming - not convenient loading into a vehicle - incomplete use of the carrying capacity of the vehicle |
| Mechanized | job-dedicated machinery |  | - high performance - saving time - convenience when loading - full use of the carrying capacity of the vehicle - reduction of labor intensity | - requires special equipment - does not remove drip tape at the same time - requires an additional operator - operator ergonomics is not respected - high cost of equipment |
|  | Without job-dedicated machinery |  | - improves working conditions for working personnel, which increases productivity - does not require special equipment | - incomplete removal of the mulch film - not convenient loading onto a vehicle - incomplete use of the carrying capacity of the vehicle - environmental pollution |

Table 3.4 - Methods for cleaning mulch film and drip tape

| Methods for cleaning mulch film and drip tape |  |  | Advantage | Drawbacks |
| --- | --- | --- | --- | --- |
| Without mechanization (hand-held) |  |  | - does not require mechanized and other means | - poor efficiency - high labor intensity of the process - high time consuming - not convenient loading onto a vehicle - incomplete use of the carrying capacity of the vehicle |
| Partially mechanized |  |  | - improves working conditions by employers, which improves performance | - requires special equipment - high labor intensity of the process - high time consuming - not convenient loading onto a vehicle - incomplete use of the carrying capacity of the vehicle, due to a loose loading |
| Mechanized | Non-specialized technique |  | - improves working conditions for operations staff, - does not require special equipment | - incomplete removal of mulching film and drip tape - not convenient loading onto a vehicle - incomplete use of the carrying capacity of the vehicle - environmental pollution |
|  | Special machines |  | - high performance - saving time - ease of loading - full use of the carrying capacity of the vehicle, due to dense packing - reduction of labor intensity | - requires special equipment - winding mulching film and drip tape on one drum - requires an additional operator - operator ergonomics is not respected - high cost of equipment |

Methods for removing drip irrigation belts provide for manual and mechanized. Mechanized methods can be when the unit is moving or in a stationary position.

Mulch removal methods include: manual; partially mechanized; mechanized (specialized equipment or non-specialized equipment); using a firing device for burning the remains of vegetation and polymer materials.

Cleaning of mulching film and drip irrigation tape from soil surface during joint harvesting involves the following methods: not mechanized; semi-mechanized; mechanized. Manual (non-mechanized) methods of harvesting when using on the large area are very time consuming. With a partially mechanized method, you also have to use manual labor. The mechanized method has not been fully brought to its final state. It also uses an additional operator to control the winding operating modes by visual inspection and controlling at moving of unit. Either these units are very expensive (Figure 1.2). The cost of existing units varies in a wide range from 1046150 to 10226650 tenge when converted into national currency. In Kazakhstan, even wealthy commodity producers are not able to purchase such units. Therefore, they prefer the use of manual labor, which takes high costs of physical labor and time.

Figure 1.2 - Comparative indicators for the cost of units

1.6 Analysis of the studies studied, setting the goal and objectives of the study to remove polymer residues from the field surface

A review of research and development and best practice experience shows that the efficiency of vegetable production is measured by the yield and energy consumption of the operation. The most significant effect on these indicators is the preservation of moisture in the soil and the reduction of weeds during the growing period. These indicators, in turn, depend on the cultivation technology. One of the most effective cultivation methods at present is the use of mulch to cover the soil surface with various materials, including plastic film and the use of flexible tapes for drip irrigation [90, 91, 92].

However, after the harvesting period, the used polyethylene mulch and flexible tape creates certain inconveniences for soil cultivation. Transparent polyethylene must decompose when exposed to direct sunlight because it does not contain soot. The stems of the plant during the period of growing vegetables create a sufficient condition for the polyethylene to be in the shade. As a result, the polyethylene remains safe and has sufficient strength. Short days in the fall are not able to affect the plastic film to reach the final decomposition stage. A study by Wageningen University scientists on the use of plastic mulching in soil shows that plastic residues in soil can accumulate and lead to serious consequences of agroecosystems. As an alternative, biodegradable plastic films are a promising product for minimizing the build-up of plastic debris and reducing soil contamination. However, the effect of residues from traditional and biodegradable plastic films on the soil-plant system is not well understood. Studies have shown that the effect of macro- and microparticles of low-density polyethylene and biodegradable plastic mulching films on the bacterial communities of the rhizosphere, profiles of volatiles of the rhizosphere and the chemical properties of the soil. This study provides evidence for both biotic and abiotic effects of plastic residues on the soil-plant system, indicating an urgent need for additional research to study their ecological effects on agroecosystems [93, 94].

Thus, self-dissolving polyethylene mulch films have not been studied sufficiently and, when mixed with soil, they create certain hazards. Therefore, it is advisable to use a non-degradable mulch film and it is necessary in the post-harvest period mandatory harvesting from surface of soil. Considering the laboriousness of removal [35] and when using a large area, a mechanized method of removing film and flexible irrigation belts is necessary. Otherwise, the remnants of polymer materials create inconvenience for soil cultivation, remaining for a long time gradually decompose.

Based on the study and analysis of the known design of technical means for removing the mulching film, the following disadvantages can be noted, which must be taken into account when developing the design diagram of the proposed device for removing mulching film and flexible drip irrigation tapes:

- the complexity of the process of removing the wound coil from the unit;

- the complexity of the design and the high cost of equipment;

- performing the operation by repeated passes with different types of machines, which increases the cost of technology;

- the complexity of loading and transporting the harvested material, due to poor-quality collection;

- maintenance of the unit by an additional operator, which does not meet safety and ergonomic requirements;

-developments do not provide for the removal of the film when the stems of plants germinate through the mulch;

- separate devices are not suitable for cleaning different widths of protective films.

Taking into account the modern requirement of vegetable production, a device is proposed to remove used mulch and flexible irrigation belts while simultaneously removing plant waste to create natural mulch in the fall, instead of removed mulch film.

Increasing efficiency and expanding functionality is achieved due to the fact that the device has an increased productivity of winding film and flexible irrigation belts by automating individual operations and to exclude the operator to regulate the winding process on the unit (Appendix W).

The purpose of this work is to increase the efficiency of soil mulching and drip irrigation by developing mechanized means of removing them from the field surface in the post-harvest period. To achieve the goal, according to the project program, the following tasks and ways of solving them were set:

- Develop technology and technical means for the removal of used polymer waste (mulch film and flexible irrigation belts) in the post-harvest period from the surface of the field.

The technology for removing polymer residues includes: operations for freeing the edge of the film from the soil; cutting off plant stems protruding through the mulch; feeding mulch and vegetable waste to the conveyor; cleaning mulch from soil and stems materials; and winding a mulch film; winding a flexible irrigation tape on a drum with the help of a distributor. All operations are mechanized by creating a mounted universal technical means aggregated by a tractor.

- To theoretically and experimentally substantiate the modes of operation and parameters of the mechanisms performing the operations of the technical means for removing the mulching film, flexible irrigation tapes. The modes of operation were the conditions for performing the operation of the technology (movement of the unit, movement and winding of materials). The substantiated parameters were the constructive and kinematic indicators of individual mechanisms by means of calculations and experiments.

- To carry out separately laboratory tests of each operation mechanisms proposed technology and the technical means for removal of mulching films and flexible irrigation tape coexistence and separate cleaning. Laboratory studies were carried out on a soil channel with separate mechanisms of the developed technical means.

- Develop the design of a device for removing used mulching film and flexible irrigation tape. Based on the results of the research, a technical task was drawn up and not based on it, design documentation was developed and an experimental sample of a experimental device was made for removing polymer residues from surface field. The technical equipment includes a list of standard and non-standard components according to the developed design documentation.

- To test the developed experimental device for removing mulch film and flexible irrigation tape in the field. Test work included preparation and production testing of the new equipment mulch film remover and flexible irrigation tape. The testing of a new sample of the device was carried out in the Almaty region. The test was carried out by a group of developers with the participation of employees of the enterprise. The test results were a test reports in accordance with the state standard on the appropriate test method for agricultural machinery and equipment.

- To evaluate the technical and economic efficiency of using the technology for removing mulch film and flexible irrigation tape. The assessment was carried out in comparison with a foreign analogue. A monograph was published based on the results of research on the project topic. Taking into account the test results, technical documentation has been prepared to create a production model of the technical means.

2 Justification of parameters and operating modes of the device for removing mulching film and flexible irrigation tapes

2.1 Features of cleaning mulch film and flexible irrigation tapes on the proposed device

Removing the mulch film after the harvesting period is one of the labor-intensive processes of up to 64 man-hour / ha manually. The plastic film used as a mulching means cannot decompose from the influence of sunlight due to the creation of a shadow effect from the plant stems. Considering the long process of decomposition of polymer residues inside the soil, certain inconveniences are created for the use of the soil (cultivation, planting, watering, etc.) [95, 96]. Taking into account the difficulties arising during the removal of mulching film and flexible irrigation belts, a technology and a means have been developed for the following operations: mowing plant stems protruding through the film opening (creating a shadow effect); cleaning the edge of the film from soil or digging in; supply of mulching film and flexible irrigation tapes to winding drums; removal of the mowed mass of plants.

For the implementation of the above operations, a hinged device for mechanized harvesting of mulching film and flexible irrigation belts was proposed (Figure 2.1) [96, 97].

1 - frame with support wheels; 2 - blade; 3 - segment mechanism for cutting plants; 4 - reducer; 5 - inclined conveyor; 6 - mulching film; 7 - reel tape for drip irrigation; 8 - hydraulic motor; 9 - belt drive; 10 - layout mechanism; 11 - rewinder for mulching film; 12 - drip irrigation tape

Figure 2.1 - Device for mechanized harvesting of mulch film and flexible irrigation tapes

The device in working condition is supported by two support wheels 1 and the hitch of the tractor. When transporting the device, the hitch is lifted and all working parts are lifted off the ground. In the working position, the support wheels of the device, like that of a towing tractor, move along the edge of the mulch strip, the cutting unit, due to the lowering of the tractor's hitch, rests on the mulching surface with shoes. When moving, the protruding stems of plants above the mulch are mowed using the mowing mechanism 3, driven by the PTO of the tractor connected through the gearbox 4. The edges of the mulching film after mowing the plant mass are removed from the soil by dumps 2. The mowed mass of plant waste, together with the mulching film 6, moves along the inclined conveyor 5, is dropped onto the field surface, while the speed of the mowed mass and the film has zero value along the horizontal plane. The removed mulch film after the inclined conveyor is wound on the drum of the film winder 11, and the tape, passing through the layout mechanism 10, is wound on the tape winder 7, driven by belts 9 from a hydraulic drive (hydraulic motor) 8. The inclined conveyor also takes the drive from the hydraulic drive, like the winding drum.

Thus, the proposed device for winding a polyethylene film allows you to expand the functionality due to the use of a tapered drive device of the film winding drum. In this case, the winding roller of the film is used as a winding drum when spreading the mulching film, which is placed between the cones of the device. After the end of winding the film on the drum, the tractor operator can dump the packed roll with the core in a convenient place by moving the lever. The lever is connected to the release mechanism using a cable. When acting on the dumping mechanism, the conical clamps of the drum open and the automatically filled roll is dumped without the participation of an additional operator (trailer). The flexible tape on the drum is also removed from the ribbon rewinder shaft using a clamping mechanism.

A means of removing mulch and flexible irrigation belts, in the post-harvest period, provides mowing of plant waste that protrudes above the mulch and covers the surface of the mulch field [32].

The machine, moving along the track, cuts off the protruding plant stems on the surface of the plastic film. In this case, the mechanisms wind the freed film from overgrowth on a transversely mounted rotating shaft. At the time of winding the film, the mown plant waste spreads after the machine to the soil surface, like natural mulch in autumn. This will ensure that moisture in the soil is retained and cut plant waste will decompose faster.

With separate harvesting of mulching film and flexible irrigation belts, this technical device can be dismantled and operated separately together with the tractor.

2.2 Calculation of the pulling forces of the unit for winding mulch film

The theoretical substantiation of the design parameters of the device for removing the mulching film should be carried out initially with the calculation of the traction forces of the unit. This is necessary to determine the traction class of the tractor, which together with the device creates the machine. To do this, apply all external forces to the unit. According to Figure 2.2, the diagram of the effect of forces on the unit looks like a combination of the following forces:

- Gм - force from the mass of the device;

- Rдм - resistance force of the film edge cleaning mechanism;

- Fмк is the resistance force of the plant stem mowing mechanism;

- Fок - resistance force of the support wheel.

The following soil reactions act to the mechanism for freeing the edge of the film from the soil: Rx, Rу, Rz. The indicated components of forces depend on the soil properties, the angle of attack of the working body and the speed of the unit movement (Figure 2.3) [98, 99].

Figure 2.2 - Scheme of the effect of forces on the device

Figure 2.3 - Scheme of the reaction from the soil to the disk

To calculate the traction resistance of the disc when deepening by the value h2 into the soil with the specific resistance k, the longitudinal component is equal to

Rx=k∙S,                                                           (2.1)

where S is the area of the active part of the disk [99, 100].

The area of the active part of the disc is defined as the area of the segment

S=R22π2180sinα,                                                   (2.2)

where R - the radius of the disc;

α - the arc angle of the segment.

The pulling force for soil removal will be equal, taking into account the number of working bodies

Rдм=Rx∙n,                                                   (2.3)

where n - the number of the removing working bodies.

The resistance of the mowing mechanism is mainly due to the friction of the support legs located at the edges of the bar. Since the support legs of the bar are placed symmetrically relative to the longitudinal axis of the unit, then the resistance of the cutting mechanism depends on the weight of the bar and the coefficient of friction.

Fмк=Pмк∙f.                                                     (2.4)

The resistance force of the support wheel depends on the load acting on the support wheel and the coefficient of friction on the rolling of the wheel

Fок=Gм∙μкач,                                                   (2.5)

where Gм- the weight of the device;

μкач - rolling friction coefficient.

Then the total traction resistance of the device is defined as the sum of the resistance components on the device [101].

Rм=Rдм+Fмк+Fок,                                            (2.6)

The determination of the tensile force of the mulch film during the removal by winding was carried out according to the design diagram of the mechanism shown in Figure 2.4. The value of the tensile force of the film was determined for conducting in order to determine the ultimate tensile strength of the polyethylene film by experimental study. These values are necessary to create a winding mechanism operating with a certain film tension. For this purpose, a safety clutch is provided on the winding drum shaft.

Figure 2.4 - Design diagram of the winding mechanism for mulching film and flexible irrigation tapes

2.3 Determination of the tensile force of the mulch film during winding on a drum

Considering that a weighty tape is wound onto the drum by gravity Q on an inclined plane with the total force P (due to the pressure of the film with plant residues and soil residues) on the conveyor, and at the same time the total force of normal reactions R appears on the horizontal section (Figure 2.5). After the horizontal section, the remains of plant and soil are separated, the film becomes almost weightless. In this case, the sliding friction force Fтс appears on the inclined surface.

Figure 2.5 - Scheme of the effect of forces in the process of winding

To solve the problem under consideration, the condition was assumed that the coil performs rotational motion. At the initial moment, the system is at rest. It was necessary to determine the tension force Fнат, which tends to break the tape. When the system passes the path l, the belt speed is balanced and will be ν = const, respectively, the angular speed of the drum will also be constant (wб=const). The equation for the change in the kinetic energy of the system

k=1nT-k=1nT0=k=1nAFke+k=1nAFki.                  (2.7)

Since, the material system was considered, the sum of the work of internal forces is equal to zero, and therefore equation (2.7) can be written in this form.

k=1nT-k=1nT0=k=1nAFke.                                       (2.8)

By the condition of the problem k=1nT0. The kinetic energy of the system at a certain moment of time when the speed of the tape is equal to ν and it moves along the plane at a distance l, then the left side of equation (2.8), after substituting the values of the components, is written as the sum of the kinetic energy.

k=13Tk=Pv22+Gv24g+Qv24g=v22gP+G2+Q2.                           (2.9)

Further, the total work from all forces and moments was determined by the following formula.

k=1nAk=lMврr-Psinα-fPcosα.                             (2.10)

After substituting all values and solving the equation, the drum torque was determined

Мвр=ω2∙r2glP+G+Q2+P∙rsinα+fcosα .                  (2.11)

After canceling by radius r, an equation is obtained that describes the tangential force in the following form

F= ω22glP+G+Q2+P∙sinα+fcosα.                              (2.12)

Thus, the obtained tangential force in equation (2.12) is equivalent to the film tension force. This force increases in a quadratic mode depending the angular velocity of the drum.

2.4 Calculation of the hydraulic drive mechanisms for winding mulch film and drip irrigation tape

The drives of the mechanisms for winding the mulching film and the drip irrigation tape were taken from the hydraulic motor, since the receipt of torque from the support wheels does not provide the required speed. This is due to the appearance of sliding of the support wheels when passing the unevenness and at the moment of turning one of the support wheels stops. As a result, a deceleration of the drum speed can occur and the tension of the wound material is released. Since both drums operate in parallel, a simplified hydraulic drive scheme can be represented in the following form (Figure 2.6).

Figure 2.6 - Simplified diagram of the hydraulic drive mechanisms for winding mulch film and flexible drip irrigation tapes

Pump 1 delivers working fluid from hydraulic tank 2 through throttle 3 to two hydraulic motors 4 and 5 and from them through filter 6 back to hydraulic tank. The output shafts of the hydraulic motors through mechanical drivers are connected to the pulleys 8 on which the belt drives act.

We took the separate parameters, according to the diagram (Figure 2.6) from the condition of the technical characteristics of the tractor that moves the vehicle, such as the length and diameter of the main line (li, di), the diameter of the drive pulleys (D), the tangential force to the pulleys (Fi) and the parameters of the hydraulic system pump tractor (maximum working volume - Wн.max, shaft rotation speed - nн., volumetric leakage factor - Koн., mechanical efficiency - ηмн.), pump delivery regulator parameters (setting pressure - pн.min). The following were selected accordingly: filter resistance coefficient (ξф); throttle parameters (Sдр., μдр.), hydraulic motor parameters (working volume - Wг, volumetric leakage coefficient - Koн., mechanical efficiency - ηмн.). The parameters of working fluids were selected according to technical requirements: kinematic viscosity (ϑ), density (ρ).

Further, an equivalent circuit was drawn up according to Figure 2.7 of the hydraulic drive for the calculation [102, 103]. The design diagram of the hydraulic drive is a complex pipeline with a series-parallel connection of individual sections (simple pipelines) 1, 2, 3 and 4.

Figure 2.7 - A variant of the equivalent circuit of the hydraulic drive of the mechanisms for winding the mulching film and flexible irrigation tapes for drip irrigation

The calculation was realized according to the following sequence:

- selection of the scale and construction of the characteristics of the pumping unit;

- drawing up the equations of characteristics for each simple pipeline included in the connection, and determining their coefficients;

- building the characteristics of simple pipelines and obtaining the total characteristics of the entire complex pipeline;

- determination of the working point, performing additional graphical constructions and analytical operations.

Taking into account the linearity of the characteristics of the pump and the

pumping unit with the pump displacement regulator, each of them was constructed using two points.

Following equations set-up of simple pipelines   characteristics are accepted for sections: 1 and 4 - turbulently; 2 and 3 - laminar. The characteristic equations for sections 1, 2, 3 and 4 are respectively constructed in this way.

∆p1=∆pтр1+∆pдр;                                             (2.13)

∆p2=2∆pтр2+∆pг1;                                            (2.14)

∆p3=∆pтр3+∆pг2;                                             (2.15)

∆p4=∆pтр4+∆pф.                                             (2.16)

Based on the data obtained, graphical characteristics are built: ∆p2=φ1(Q); ∆p3=φ2(Q); ∆p1,4=φ3(Q).

According to the rules of graphical addition of the characteristics of parallel sections (along the Q axis), the total characteristics of sections 2 and 3 (line 2,3) were obtained.

At the next stage, the obtained characteristics summed up graphically (line ∑2,3) with the characteristic ∆p1,4=φ3(Q) along the p axis, and as a result, the total characteristic was obtained of the entire pipeline (line Σ).

The intersection of the obtained characteristic of a complex pipeline with the characteristic of the pumping unit determined the operating point of the hydraulic system (point R).

Figure 2.8 - Characteristics of a hydraulic drive pump for winding mechanisms for mulching film and flexible irrigation tapes for drip irrigation, obtained by connecting points A and A'

As a result, the following was determined: tangential speed (v1 and v2) to the pulleys of the drive shafts of the winding devices (mulching film, flexible irrigation belts or an inclined conveyor); power consumed by the hydraulic drive (N); efficiency of the hydraulic drive (η).

The data obtained made it possible to determine analytically the main parameters of the hydraulic drive. As a result, a hydraulic drive was installed for the developed device with appropriate parameters. To check up a more realistic picture of the indicators, experimental studies were carried out at the time of testing the unit as a whole. Experimental data are presented in sections 3 and 4.

The strength calculation of the PVC pipe core intended for winding during cleaning of mulch film was carried out using the Inventor Professional 2018 software. The flexural stress of a PVC pipe, taking into account the elasticity of polymer films and soil resistance during winding, obtained a pipe displacement (up to 451 mm) from its central axis. This displacement is accepted as permissible to maintain a steady state of the pipe between the pressure cones (Appendix G).

3 The experimental studies of the influence of factors on the technology of moving off of mulch film and flexible irrigation tape

3.1 Work program

The work program provided for:

- determination of the physical and mechanical properties of the materials plant origin;

- determination of changes in the strength indicators of polymer films from various factors;

- determination of soil resistance when winding film;

- examination of the process of winding polymeric materials.

3.2 Research methodology

Laboratory studies of the influence of factors on the technology of harvesting mulch film and flexible irrigation belts were carried out according to the presented research infogram (Figure 3.1).

Figure 3.1 - Infographics of laboratory research

The angles of friction on a steel surface without pressure at rest and in motion were determined on a specially designed device (Figure D.1).

The material under study was freely placed on the surface of a steel sheet in a horizontal position. The changed position of the sheet, the moment of the beginning of the movement of the material, was fixed, and the angle of its inclination was measured using the sector mechanism (Appendix D). The temperature and humidity of the test material in all experiments was maintained within the following limits: temperature 298 + 1K, humidity 10 + 2% [104].

The coefficient of friction at low pressures (in the range of 500 ... 1500 Pa) was determined using a setup developed by employees [105]. The material under investigation is poured into a box on a drum, the latter is driven into rotation by a geared motor. During the operation of the device, the box, hinged on the axis, due to the adhesion forces between the drum rim and the test material, tends to turn in the direction of the drum rotation, exerting pressure through the support plates on the bracket of equal resistance with the sensors. According to the readings of the millivoltmeter according to the calibration schedule, the true value of the forces on the movement of materials of plant origin was determined. Friction coefficients were calculated using the formulas shown in Figure 3.1.

Determination of changes in the strength parameters of polymer films was carried out under natural conditions under the influence of the duration of sunlight [106, 107, 108]. During the tests, the samples were fixed on the soil surface located in an open area (Appendix D). Changes in appearance, color, formation of cracks and other surface defects of the samples were periodically noted, and their strength indicators were also determined [109,110,111].

Polymer films were tested in accordance with the requirements of GOST 14236-81 [111]. To test polymer films, samples in the form of a rectangle 25 mm wide and at least 150 mm long were used (Figure 3.5) [112, 113]. The tests were carried out at a temperature of 23 ± 2° С and a relative humidity of 50 ± 5%. The speed of the extension of the clamps of the testing machine is 25 mm / min. During testing, the load and elongation of the sample were measured continuously. When recording "load - elongation", the tensile test parameters were determined. Samples that failed during testing outside the calculated length, or in which material defects were found during testing, were not taken into account.

To determine the soil resistance [114] during winding, an installation for winding a mulch film was used, which consists of a frame-stand, a shaft with a winding sleeve and a torque wrench. A mulching film with perforated holes 500 mm wide and 100 μm thick was wound onto the reel of the installation, with the edges embedded in the soil. The measurement of the torque was carried out with a torque wrench with overcoming root residues with cut stems and without them. The measurement was carried out at the planting sites under the mulch film of tomatoes and peppers. The cut height of the stems was 80 - 100 mm (Appendix D).

An experimental study of the process of winding polymer materials was carried out on a developed laboratory bench including a soil channel, trolleys, a drive for the trolley and winding devices, an unwinding drum, and a control panel. The working width of the winding bobbin is 0.4 m. The change in length between the winding device and the unwinding drum was carried out by moving the carriage up to 6 meters (Figure 3.2). Since one of the requirements of the technology when winding rolls is to ensure a constant tension, both at the steady-state speed and in the transient mode [115], the degree of roll compaction was estimated by the coefficient. The coefficient of relative compaction of a polyethylene film (кrel) from the diameter of the wound roll was determined by the following formula as the ratio of the diameter of the factory roll (D0) to the diameter of the used film rewound from the field (D1)

Figure 3.2 - Scheme and general view of a laboratory stand for determining the degree of compaction of a plastic film

3.3 Research results

The results of determining the angles of repose and the coefficients of friction of plant waste showed the following data. The internal angle of friction on the polyethylene surface without pressure for the stems of tomato, bell pepper, and eggplant was respectively α = 44, α = 42, α = 40 degrees. The dependence of the friction coefficient of plant raw materials on normal pressure within the experiment was linear [116]. This phenomenon is explained by the fact that with an increase in normal pressure, a part of free moisture is not squeezed out onto the friction surface, due to insufficient pressure, and friction increases evenly.

The results of the study on tensile and breaking strength, relative elongation at maximum load and relative elongation at break were graphically interpreted in the form of dependencies [108, 116], the values ​​of which are given in Table D.1 (Appendix D). The durability of polyethylene films in natural atmospheric conditions varies dramatically. The results of studying the patterns of changes in strength properties on specimens tested for 105 days indicate that the tensile strength (σz) did not change significantly for all specimens. Also, in almost all samples, the tensile strength (σr) also did not change significantly, except for the 30 μm sample, it decreased by 47%. Elongation at maximum load (εz) decreased at the maximum at 120 μm (black) by 82% and at the minimum at 80 μm by 67%. Elongation at break (σr) decreased maximally for 120 μm (black) by 82% and minimally for 50 μm by 47%.

Determination of soil resistance when winding the film on a reel in places with

root residues with cut tomato and pepper stems, as well as in places without  plant

residues, is presented in Table 3.2.

Table 3.2 - Soil resistance when winding

| Measurement number | Soil resistance without plant residues, Nm | Soil resistance with tomato plant residues, Nm | Soil resistance with plant residues of pepper, Nm |
| --- | --- | --- | --- |
| 1 | 14,7 | 22 | 22 |
| 2 | 12,5 | 29,4 | 23,5 |
| 3 | 16,9 | 41,9 | 20,6 |
| 4 | 16,9 | 30,9 | 27,9 |
| 5 | 15,4 | 31,6 | 25,7 |
| Measurement average | 15,2 | 31,1 | 23,9 |

As a result of the conducted studies of the process of winding polymeric materials, the dependences on the applied force of the diameter of the roll and the coefficient of relative compaction presented in Figures 3.3 were obtained. From the analysis of the obtained dependences it can be seen that with small changes in the film tension forces in the initial state, the dependence is not linear. With the application of an increase in the value of the strain, the dependencies become linear. This is due to the presence of damage to the film, which is wrinkled to form voids between the layers of the film.

The index of the degree of compaction of the Кот is maximally near the mark of 0.9, this limitation is due to the fact that there is no ideal condition for a new film under factory conditions. In addition, the resulting deformations and breaks do not give a tight fit of the layers.

Figure 3.3 - Dependence of the change in the roll diameter and the coefficient of relative compaction of the polyethylene film on the value of the tensile force of the polyethylene film

4 Development of an experimental model of a device for cleaning of polymer residues, field test and estimation of economic efficiency

4.1 Program of experimental work

To develop an experimental prototype of a device for cleaning polymer residues, the following work program was drawn up:

-conducting patent information research;

- preparation of technical specifications for the development of design documentation for the manufacture of an experimental sample of a device for removing polymer residues from the field;

-development of design documentation for the manufacture of an experimental sample of a device for removing polymer residues from the surface of field;

-production of an experimental sample of a device for removing polymer residues from the field;

-field testing of the developed device in the conditions of the farm;

- assessment of the economic efficiency of the developed device.

4.2 Methodology for performing device development, field testing and cost-benefit of the economic efficiency

4.2.1 Conducting patent information research

To develop a new design of the device [117], preliminary patent research was carried out on materials from the leading countries of the CIS and the USA, Canada, Great Britain, France, Germany, China, Italy).

Patent and technical literature, domestic journals, works of institutes, descriptions of patents and copyright certificates for the current period were studied in order to determine the tendency in the development of technical means of mechanization for removing mulch film and flexible irrigation tapes after harvest. The prototypes and analogues of the device under development were determined. Applications were submitted for obtaining a patent for inventions of the Republic of Kazakhstan. The patent research report was drawn up in accordance with GOST R.15.011-96 [117, 118, 119].

4.2.2 Completing up technical specifications for the development of design documentation

Terms of reference for the development of design documentation (Appendix E) for the manufacture of an experimental model of a device for removing polyethylene film from the surface field were drawn up in accordance with the requirements of the standards for the supply of products to production [119].

4.2.3 Development of design documentation for the manufacture of an experimental model of a device for removing film from the surface field

The design documentation for the manufacture of an experimental sample of a device for removing film from the surface field was developed by the performers of this project. The technical documentation was carried out in accordance with GOST using the AutoCAD program in accordance with the compiled technical specifications for these products [120, 121, 122].

4.2.4 Production of experimental sample device

An experimental sample of a device for removing film from the surface field was manufactured in accordance with the developed design documentation in the IP "MOLDIR" company in accordance with the contract concluded between the Kazakh National Agrarian University. During the manufacturing period, technical control was carried out on the progress of work by visiting the site by the project executors on a weekly basis, and the status of the contract was discussed, the errors identified were finalized (Appendix Zh).

4.2.5 Field test of the developed device under farm conditions

To carry out a production test of an experimental sample of a device for cleaning films and flexible drip irrigation tapes, the following stages of work were performed:

- preparation of a field for laying a flexible irrigation tape for drip irrigation, mulching the soil, planting seedlings;

- laying flexible irrigation tape for drip irrigation, mulching the soil with plastic wrap, planting seedlings of vegetable crops in the conditions of the farm using a unit for semi-automatic planting;

- study of the state of the field where the seedlings were planted;

- preparation of an experimental sample of a device for cleaning films and flexible drip irrigation tapes for production testing;

- production tests of the device for cleaning films and flexible drip irrigation tapes.

All of the above works were carried out according to the agreement of the university with FE "ZAGROUP" Kilybaev Almas Dauletbekovich (at the production site) in the village of Almaly Enbekshi, Kazakh district, Almaty region.

The necessary conditions for the preparation of the field were the preparation of the field (plowing, clearing the field from foreign objects and harrowing). These operations were performed to ensure high-quality mulching when planting seedlings of vegetable crops. Preparation of the soil for planting tomato was carried out in deep (27–30 cm) autumn plowing, early spring harrowing and, before planting loosening with moldboard tools, followed by harrowing. When assessing the characteristics of the site, the type of soil, the name of the soil by its texture, relief, microrelief, moisture and soil hardness were determined according to GOST 20915 [123-132].

Operations for laying a flexible tape of drip irrigation, mulching the soil and planting seedlings were carried out on the irrigated field of FE "ZAGROUP" using the development of the research group [133]. The field length of each test site was 200 m, the total area for control work was 2 hectares for tomato. At the end of the site, work was carried out on filling with polyethylene for mulching the soil and water for pre-planting irrigation. The MTZ-50A wheeled tractor in good condition was used as a traction force. The length of a standard roll for mulching was 250 meters and a width of 1.5 meters, a spare roll of film was installed in a special place on the machine. The thickness of the mulch film was 30 microns. For planting, seedlings without briquettes were used, since the unit provides for pre-planting irrigation in the holes. Therefore, the seedlings enter a moistened hole and completely retains their vitality until water is supplied for irrigation. Operators for planting seedlings were seated in a special seat, and in front of them was a box with seedlings. As needed, they themselves took the required number of seedlings.

The preparation of an experimental sample of a device for collecting films and flexible drip irrigation tapes for production testing was carried out in a laboratory (Appendix I). For this, the device was fully equipped with all mechanisms, including a developed block for measuring parameters (unit speed, power consumption of driving mechanisms), previously tested on laboratory benches. The hydraulic motors designed to drive the inclined conveyor, drums for winding mulch and flexible drip irrigation tapes were tested. The force of the springs on the friction mechanisms of the drive of the winding drums and the actuation of the release mechanism of the roll were checked. Hand rotation of the inclined conveyor rollers, fasteners and tire pressures were tested. After performing the verification work, the device was carefully dismantled for transportation to the test site [134]. The transportation was carried out on a vehicle equipped with a manipulator for loading and unloading with a carrying capacity of up to 5 tons. Then the device was repeatedly re-equipped. After the transported device to the test site, the equipment was unloaded on a flat area. Disassembled mechanisms were completed, all connections were subjected to verification. To perform the operation, the unit was connected to an MTZ-50L wheeled tractor, connected:

- power take-off shaft of the tractor to the drive of the mowing mechanism of the unit;

- the hydraulic system of the tractor was connected to the hydraulic system of the hydraulic drive of the drums and the inclined conveyor of the unit;

- the rope release mechanism for the roll was connected to the operator's cab.

The study of the state of the field where the seedlings were planted included: determination of soil hardness using a static penetrometer PSG-MG 4; determination of soil moisture.

When testing the unit for collecting film and flexible drip irrigation tapes, the quality indicators of the technological processes were determined. The technological processes include four operations: mowing the stems, digging out the longitudinal edges of the film from the soil, feeding mulch and remainder crop with an inclined conveyor, winding mulching film and flexible irrigation tape on drums. The agrotechnical assessment was carried out when the machine had been running for at least 10 hours of main work.

The tests were carried out in two ways: joint harvesting of mulch film and flexible irrigation tapes; winding of flexible drip irrigation tapes in the absence of a mulching film separately.

Before sampling, the machine was set to optimal adjustments to suit test conditions and specifications for harvesting mulch film and flexible drip belts.

The machine was tested in the modes recommended by the operating instructions. The optimal operating mode is the one that ensures the removal of the film without tearing, in the form of a tightly wound material into a roll on a drum (Figure 4.1).

Figure 4.1- Fragments of the test of the device for assembling mulch film and flexible irrigation belts together and separately

4.2.6 Assessment of economic efficiency of development

For a comparative assessment, only mechanized disposal methods were selected. As a basic variant, the performance index of the ROCCA company machine PMR-01 for removal mulching film made in the USA was used [135]. Evaluation of the economic efficiency of the proposed experimental sample for the removal of mulch film and flexible drip irrigation belts were carried out in accordance with the interstate standard GOST 34393-2018 “Agricultural machinery. Methods of economic evaluation "(Appendix L) [136, 137, 138].

4.3 Results of work

4.3.1 Patent-information search

According to the results of the patent study, the shortcomings of the previously obtained: patent of the Republic of Kazakhstan for the utility model "Device for removing mulch film" IPC А01G 13/02 (15.11.2016, bull. No. 15) . An application was filed for a patent of the Republic of Kazakhstan for the invention "A device for removing and winding polyethylene films from the surface of the field" (13.09.2018, No. 18106). The patent for the invention of RK 34443 was received according to application No. 18106 (Published 03.07.2020, bulletin No. 26). An application was filed for obtaining a patent for inventions "A device for removing and winding mulching film to a machine for harvesting it from rows of plants" (Registration No. 2019 / 0890.1 dated 09.12.2019) (Appendix W).

4.3.2 Terms of reference for the development of design documentation for a device for removing polymeric materials from the surface field

The terms of reference for the development of design documentation for the manufacture of an experimental sample of the device included: name and scope, basis for development; purpose and purpose of development; sources of development; technical requirements; main indicators of the quality of the technological process. The terms of reference for the development of design documentation for the manufacture of an experimental sample of the device was approved by the scientific part of the university on August 22, 2019 (Appendix E).

4.3.3 Design documentation of a device for removing polymeric materials from the field surface

The development of design documentation for the manufacture of an experimental sample of a device for cleaning polymer residues from the surface of the field was developed on the basis of the prepared "Terms of Reference for the Development of Design Documentation" and included technical documentation consisting of a technical project, made in AutoCAD with a total volume of 340 pages in A4 format [120, 121]. The documentation included: departmental specification, assembly drawings, assembly units of units and working drawings of parts with specifications (Appendix Zh).

4.3.4 Experimental device for cleaning mulch film and flexible drip irrigation belts from the field surface

The manufactured experimental sample of the device for removing polymer residues from the field surface consists of two modules:

-film-winding module for removing the used mulch film in the post-harvest period, mowing crop residues protruding through the film, cleaning the edge of the film from soil, transporting the crop after the unit, cleaning  the film, and winding the film onto the drum;

- rewinder of flexible drip irrigation tape.

Both modules can work together or independently, separately,  on the needs of the technological process for removing polymer residues.

A manufactured sample of the device is presented in the appendix (Appendix I).

4.3.5 Results of field tests of the developed device under farm conditions

Field tests of the developed device were carried out to remove polymer residues from the surface of the field after harvesting (tomato) in FE "ZAGROUP" Kilybaev Almas Dauletbekovich (at the production site) in the village of Almaly Enbekshi, Kazakh district, Almaty region. During the tests, timing data on the operation of the unit and its mechanisms were obtained. The data were obtained by registering the parameters in the unit of the connected self-recording complex of devices on an electronic medium. The fixed parameters were: forward speed of the unit; rotation frequency of drive shafts of mulching tape winders and flexible irrigation tapes; energy consumption to drive the drive shafts; change in pressure in the hydraulic line to drive the working bodies. By a traction dynamometer, the values of the traction force were obtained for the displacement of the unit and separately for the device. The obtained parameters were processed and presented as a graphical dependence in the appendix of this report (Appendix K). The processed materials were used to evaluate the economic efficiency of the unit.

4.3.6 Assessment of economic efficiency of development

Direct operating costs of monetary fund’s attributable to the performance of a unit of operating time for new equipment and analogs amounted to 14,056.40 tenge / ha and 23,829.34 tenge / ha. The reduction in the need for machine operators and auxiliary workers is 50%. Annual savings of funds for the annual actual volume of work of new equipment in comparison with the analogue is 718 310.89 tenge. The payback period was 1.15 years (Appendix L).

CONCLUSION

1. Modern domestic and foreign experience in growing vegetables shows that mulching the soil with plastic wrap helps to preserve moisture, temperature and suppress the development of weeds under mulch. Depending on the method of irrigation, the amount of water consumed can be reduced by up to 30-50%. It is very effective to use drip irrigation systems with intensive technologies for growing agricultural crops, primarily vegetables, when the size and quality of the resulting crop largely depend on the accuracy of maintaining the moisture regime and diet. When used together, soil mulching and drip irrigation system can achieve a very efficient vegetable growing result.

2. In the post-harvest period, to prepare the field for the next planting, it is advisable to remove previously used polymeric materials for mulching and drip irrigation. The labor intensity of manual removal of used polymer residues is in the order of 64 persons × hour / ha. The cost of existing foreign means for mechanized cleaning of polymer residues varies in the range of 2200-25000 US dollars. Based on the results of the analytical review, the technology and means for the destruction of polymer waste in the post-harvest period were classified. Taking into account the shortcomings in existing technologies, a technology and means for removing polymer residues from the field surface have been proposed.

3. The design parameters and operating modes of the mechanisms of the device for removing polymer residues (mulching film and flexible irrigation belts) in the post-harvest period have been theoretically substantiated. An analytical calculation of the hydraulic drive of the mechanisms for winding mulch film and flexible tape of drip irrigation has been performed. An indicator of the stress state of a PVC pipe from the effect of the elastic force of the wound polymer film and soil resistance is obtained. The displacement of the pipe from the central axis when winding the mulch film was 451 mm. This displacement of the pipe is accepted as permissible to maintain the steady state of the pipe between the pressure cones. The results of the calculation made it possible to identify the values of the parameters of the mechanisms being developed and to select the component parts of the device with standard ones.

4. Experimentally established: the angle of friction of the plant mass to justify the angle of inclination of the conveyor; dependence of the coefficient of friction of waste of plant materials on polyethylene film; resistance of soil and germinated plants to film separation (mulch). Graphical dependences of comparative indicators (change in tensile strength, rupture and relative elongation) of polyethylene film with different thicknesses (30, 50, 80, 100, 120 microns) after changing the state from natural and climatic conditions were obtained experimentally. The dependences of the change in the diameter of the wound mulching film of the coefficient of relative compaction of the polyethylene film on the change in the tension force during winding (at the same length of the film with different values ​​of the tension force) are obtained. The obtained results of experimental studies made it possible to substantiate the parameters and operating modes of the mechanisms of the device for development.

5. On the basis of the developed technology for removing polymer residues in the post-harvesting period from the surface of the field, a means has been created for its implementation (technical task, design documentation, prototype) in the form of an experimental sample. The developed technical means for removing polymer residues from the field surface can work for joint and separate harvesting of mulching film and flexible irrigation belts.

6. The results of production tests of the developed technology and means for removing polymer residues from the surface of the field in the post-harvesting period have shown positive results, ensuring the mowing of plant waste residues, removal of polymer residues by winding on a tubular bobbin with a certain density. The average speed of the unit was 6.4 km / h, productivity was 1.48 hectares / hour. The reeled polymer waste of mulch film and flexible irrigation belts had compactly packed rolls that were convenient for loading and transportation.

7. Evaluation of the completeness of solutions to the tasks of the project consists in conducting an analytical review on this issue, theoretical and experimental studies in the development of technology and means after harvesting period of vegetables for removing polymer residues, such as mulch film and flexible polymer tapes for drip irrigation,

8. Recommendations and baseline data for the specific use of the results stem from theoretical, practical research results and field trials of a polymer remover, which opens up opportunities for promising new technologies in the mechanization of vegetables.

9. The technical and economic efficiency is assessed by production tests of the proposed technology, which makes it possible to increase the efficiency of the technology for using soil mulching and drip irrigation using flexible irrigation belts when growing vegetables and melons, to increase yields, reduce the cost of production and introduce comprehensive mechanization.

10. Evaluation of the scientific level of the work performed in comparison with the best achievements in this field is the development of a mechanized technology for the removal of polymer residues. Technical means have been developed to implement this technology, for which patents of the Republic of Kazakhstan have been obtained.

LIST OF REFERENCES

1. Goldfein M. D., Kozhevnikov N. V., Ivanov A.V., Kozhevnikova N. I., Timush L. G. Fundamentals of ecology, life safety and environmental and legal regulation of nature management. – М.: Publishing house Russian state University of trade and Economics, 2006. – 340 p. [In Russian]

2. Goldfein M. D., Zaikov G. E., Kochnev A. M. Fundamentals of a modern strategy for solving problems related to polymer contamination of the earth // Bulletin of the Kazan technological University. Vol. 17. – 2014. – № 13. – P.234–237. [In Russian]

3. Tarakanov G. I., Mukhin V. D., Shuin K. A., etc. Olericulture: textbook / revised and expanded. - Publishing house. 2-nd ed. – М.: Kolos, 2002. – 320 p. [In Russian]

4. Alshanov R. A. Kazakhstan on the world market. Potential, challenges and solutions. – Almaty.: Atamura, 2010. – 125 p. [In Russian]

5. Apsalyamov N. Trends in the development of the food industry in the Republic of Kazakhstan // Transit economy. – 2004. – № 5 (44). – P.39–46. [In Russian]

6. Kaigorodtsev A. A. Development of the food industry in Kazakhstan based on marketing principles // Al-Pari. – 2004. – № 1. – P.111–113. [In Russian]

7. Soviet encyclopedic dictionary / editor-in-chief A.M. Prokhorov. – Publishing house. 4-th ed, revised and expanded. – М.: Soviet encyclopedia, 1981. – 1600. [In Russian]

8. Litvinov S. S., Irkov I. I. Modern technologies in vegetable growing: Collection of scientific papers on vegetable growing and melon growing for the 80th anniversary of the Foundation of the all-Russian research Institute of vegetable growing of the Russian agricultural Academy. – М.: LLC «Рolygraph-Business», 2011. – P.21-31. [In Russian]

9. Astanakulov T., Baimuradov H., Nazarieva S. H. Mulching the soil increases the early harvest // Potatoes and vegetables. – 2004. – № 7. – P.5-6. [In Russian]

10. Keshubaev Z., Tleppaev A., Ayupov A., Musabekov A. Analysis of crop production in the Republic of Kazakhstan. – Almaty.: 2010. – 68 p. [In Russian]

11. Lebedeva A. T. Mulching the soil increases the yield // Potatoes and vegetables. – 2006. – № 4. – P.17–18. [In Russian]

12. Kudryashov Yu. S., Dyikanova M. E. Mulch the soil with films when growing tomatoes in unheated film greenhouses // Potatoes and vegetables. –2007. – №4. – P.21–22. [In Russian]

13 RU 2646623 C2, 06.03.2018. [In Russian]

14. RU 2065678 C1, 27.08.1996. [In Russian]

15. Khazimov Z. M., Khazimov M. Z. To justification of the parameters of the work wheel of the seedling planting device for soil mulching // Journal of the University of Chemical Technology and Metallurgy – 2011. – Vol. 46, No. 1. – P. 221-226. [In English]

16. Pre-feasibility study "Creation of production facilities for fruit and vegetable cultivation using drip irrigation technologies in various regions of the Republic of Kazakhstan». – Astana, 2009. – 71 p. [In Russian]

17. SU 1561836 A1, 07.05.1990. [In Russian]

18. RU 105111 U1, 10.06.2011. [In Russian]

19. SU 1158066 A1, 30.05.1985. [In Russian]

20. RU 2283556 C1, 20.09.2006. [In Russian]

21. Technical documentation of the company Ferrari / Construzioni meccaniche Ferrari. Ricambi FPA, 2009. – 24 p. [In English]

22. RU 18869 U1, 2001.

23. RU 2199201 C2, 27.02.2003.

24. Zits P. Kombinierte Maschiene für Folienverlegung und Setzling // Deutscher Gartenbaum – 1989. – № 33. – 43 p. [In English]

25. Kramer K.H., Estler M. Maisaubau mit Folie // Landtechnik – 1981. – № 6. – P.291–299. [In English]

26. SU 1501943 A1, 23.08.1989. [In Russian]

27. KZ 19893 A4, 2008. [In Russian]

28. KZ 24414 B, 2011. [In Russian]

29. KZ 29915 A4, 2015. [In Russian]

30. Khrabrov, M. Yu. Resource-saving technologies and technical means of irrigation: abstract of the doct. techn. sciences. – М., 2008. – 48 p. [In Russian]

31. Gil L. S., Dyachenko V. I., Pashkovsky A. I., Sulima L. T. Modern industrial production of vegetables and potatoes using drip irrigation systems. – Kiev: Ruta, 2007. – 392 p. [In Russian]

33. Safe drip irrigation systems and technologies: scientific review of the Federal state budgetary institution «Russian research Institute of land reclamation problems». – М.: FSSI CSTI «Meliovodinform», 2010. – 52 p. [In Russian]

34. Gerchuk I. Irrigation systems for vegetable crops: recommendations. - Chisinau, 2008. - 32 p. [In Russian]

35. Muratbekkyzy T., Khazimov K. M. Removal of mulch film and flexible drip irrigation belts from the field during the post-harvest period // Collection of the international scientific and practical conference of young scientists held within the framework of the «International master's summer school» – Almaty, 2018. – P.174-180. [In Russian]

36. Bremener S. M. The vitamins and their clinical application. – М.: Medicine, 1976. – 420 p. [In Russian]

37. Bromstein V. D. Chemical composition of different types of cabbage, onions and green vegetables // Canning and vegetable drying industry. – I960. № 2. – P.31–35. [In Russian]

38. Wiloch Elizabeth. Vegetables are always healthy. / Translation from German.- М.: Food industry, 1973. – 143 p. [In Russian]

39. Campbell K., Campbell T. Results of the largest study of the relationship between nutrition and health / translated from english by V. Urazaeva. – М.: The China study, 2013. – 528 p. [In Russian]

40. Zhaparov K. Zh., Chashin V. K. Food security of the Republic of Kazakhstan // Food industry. – 2004. – P.56-57. [In Russian]

41. Saveleva A.V. The role of the food problem in the modern world economy // HSE economic journal. Vol.17. – 2013. – № 3. – P.524–539. [In Russian]

42. Tleppaev A., Musabekova A., Ayupov A. Analysis of crop production in the Republic of Kazakhstan. Rating Agency of the RFCA. – Almaty: 2010. – 68 p. [In Russian]

43. Plotnikova T. V., Poznyakovsky V. M., Larina T. V., Eliseeva L. G. Examination of fresh fruits and vegetables: educational and reference guide. - Novosibirsk: Siberian University, 2001. – 302 p. [In Russian]

44. Kovalev N. G., Khailis G. A., Kovalev M. M. Agricultural materials (types, composition, properties): the journal «Agrarian science». – М.: IR «Rodnik», 1998. – 208 p. [In Russian]

45. Lomakin M. M. Mulching tillage // Agriculture. – 1985. – № 6. – P.47–49. [In Russian]

46. Potashkin K. S., Konstantinov, M. M., Gerasimenko I. V. Classification of agricultural aggregates for vertical soil mulching // News of OGAU. – 2015. – № 5. – P.95–98. [In Russian]

47. Poteshkin K. S., Konstantinov, M. M., Hmura A. N. Justification of parameters of a tillage unit for cutting water-absorbing slits with simultaneous mulching them with straw // Agroengineering science – agricultural production: collection of reports of the international scientific and practical conference dedicated to the 50th anniversary of the Celin Scientific research Institute of mechanization and electrification of agriculture. P. 2. – Kostanai. – 2012. – P.54–59. [In Russian]

48. Genelya, S. V., Gulya V. E. Polymer films for growing fruits and vegetables. – М.: Chemistry, 1981. – 231 p. [In Russian]

49. Yershova V. L., Skurtul A. G., etc. Recommendations for seedling industrial technology for the production of seedling tomatoes. – Kishinev, 1980. – 107 p. [In Russian]

50. Maslov V. A., Andreev Yu. M. The yield of early cabbage depends on the volume of the substrate and the return of seedlings // Potatoes and vegetables. – 2007. – № 4. – P.20–21. [In Russian]

51. Popova V. V. Microclimate of the field created by a polyethylene film-mulch and its influence on the formation of the potato crop in the Rostov region: abstract of the candidate of agricultural sciences: 03.20.03. - L., 1970. – 26 p. [In Russian]

52. Lomakin M. M. Mulching tillage on slopes. – М.: Agropromizdat, 1988. – 184 p. [In Russian]

53. Report on marketing research in the industry under the code oked 01.13 «Growing vegetables and melons root crops and tubers». – Almaty: – 2011. – P.22–33. [In Russian]

54. Chebotar L. G. Mulching the soil increases the yield of cucumber // Potatoes and vegetables. – 2006. – № 4. –7 p. [In Russian]

55. Kidorkina V. F., Kurtiakov etc. Early potato in the Republic of Khakassia // Potatoes and vegetables. – 2006. – № 4. – 15 p. [In Russian]

56. Cekleev G. Gradinarova. – Sofia, 1979. – 120 p. [In Russian]

57. Kayla Snyder, Christopher Murray, Bryon Wolff. Insulative effect of plastic mulch systems and comparison between the effects of different plant types // Open Agriculture 2020; Vol. 5, Issue 1 P.317–324 [In English]

58. Sirivlya A. G. Vegetables all year round. – Almaty: Kainar, 1985. – 108 p. [In Russian]

59. Sirivlya A. G. Plastic films in vegetable growing. – Almaty: Kainar, 1981. – 110 p. [In Russian]

60. Wurr D. Crop modelling in horticulture: State of the art // Scientia Horticulturae – 1981. – Vol. 56, No. 3. P.211-218. [In English]

61. Tarasc M. // Zahradnictvo. – 1982. – Vol. 7, No. 4. P.164-165. [In English]

62. Salvestrin J. // Farmers Newsletter. – 1980, No. 148. P. 30-31. [In English]

63. Verbitskaya N. M. Growing corn under light-breaking film // In: Achievement of agricultural science and practice, Ser.1. – 1983. - 57 p. [In Russian]

64. Otunchiev O. Regulation of the thermal regime of irrigated soils in Northern Kyrgyzstan: abstract of the cand. tech. sciences. – Tashkent, 1982. – 22 p. [In Russian]

65. Lebedeva A. T. You can't do without mulching // Potatoes and vegetables. – 2007. – № 4. – P.17–18. [In Russian]

66. Avrov, O. E., Moroz Z. M. Use of straw in agriculture. – L.: Kolos. Leningrad otd, 1979. – 200 p. [In Russian]

67. Sablikov M. B. Agricultural vehicles. P.2. Fundamentals of theory and technological calculation. – М.: Kolos, 1968. – 148 p. [In Russian]

68. Water resources of Kazakhstan in the new Millennium. Overview of UNDP. – Almaty. 2004. – 132 p. [In Russian]

69. Camp C.R. Subsurface drip irrigation: A review (Review) // Transactions of the American Society of Agricultural Engineers – 1998. – Vol. 41, No. 5. P.1353-1367. [In English]

70. Gafiatova T. P., Lebedova O. I. Some features of the development of the agro-industrial complex in the Russian economy // Problems of the modern economy. – Saint-Petersburg, 2011. – №1 (37). – P.14–16. [In Russian]

71. Chernenko Yu. Yu. Factors of intensification of open-ground vegetable production in Ukraine on an innovative basis // Innovations in agriculture: Problems and prospects. – Belgorod, 2015. – №1 (5). – P.30-36. [In Russian]

72. Turbin B. G. Agricultural vehicles. – М.-L.: Selhozgiz, 1957. – 366 p. [In Russian]

73. Current agricultural systems. – 2013. – № 8. – P.16-17. [In Russian]

74. Kalyada K. V., Duduka A. A. Modern technologies of cultivation of agricultural crops: recommendations. – Grodno: GGAU, 2010. – 340 p. [In Russian]

75. SU 1739899 A1, 15.06.1992. [In Russian]

76. SU 425931 A1, 30.04.1974. [In Russian]

77. BY 12159 C1, 30.08.2009. [In Russian]

78. BY 12168 C1, 30.08.2009. [In Russian]

79. Domankov V. M., Ageichik V. A., Polobok V. N. Justification of parameters of the film winding mechanism // Mechanization and automation of agricultural production. Issue.1. – Minsk: Uraji, 1987. – P.104-106. [In Russian]

80. SU 718054 A1, 29.02.1980. [In Russian]

81. US 5386876 A, 07.02.1995. [In English]

82. US 8302699 B2, 06.11.2012. [In English]

83. SU 1653639 A1, 07.06.1991. [In Russian]

84. CN 103749015 B, 27.01.2014. [In English]

85. KZ 1782 U, 15.11.2016. [In Russian]

86. Vysotsky A. A. Dynamometry of agricultural machines. – М.: Engineering, 1968. – 290 p. [In Russian]

87. OST 10.2.2-2002 Testing of agricultural machinery. Energy assessment methods: Introduced 2003-03-03. – М., Gosstandart of Russia, 2003. – 16 p. [In Russian]

88. OST 10.4.2-2001 Testing of agricultural machinery. Machines and tools for surface and shallow tillage. Methods for evaluating functional indicators: Introduced 2002-03-01. – М., Gosstandart of Russia, 2002. – 14 p. [In Russian]

89. Gamzatova B. A. Intensification of production from the crisis // Bulletin of the Altai state agrarian University. – Barnaul, 2013. – № 9 (107). – P.114–116. [In Russian]

90. Hluzov N. T. Some methodological approaches to the development of a complex of machines for energy-saving technologies of tillage and seeding // Achievements of science and technology in agriculture. – 2010. – № 10. – 56 p. [In Russian]

91. Mudrova A. A., Mudrov A. G. Energy saving issues // KazSAU Bulletin. – 2012. – № 1 (23). – P.98–101. [In Russian]

92. Machnev A.V. Energy-saving technology and technical means of subsurface-scattered sowing of grain crops: autorefereat dis. doc. techn. sciences. – Penza, 2011. – 39 p. [In Russian]

93. Yueling Qi, Xiaomei Yang, Amalia Mejia Pelaez, Esperanza Huerta Lwanga, Nicolas Beriot, Henny Gertsen, Paolina Garbeva, Violette Geissen. Macro- and micro- plastics in soil-plant system: Effects of plastic mulch film residues on wheat (Triticum aestivum) growth // Science of the Total Environment – 2018. – Vol. 645, P.1048-1056. [In English]

94. Yueling Qi, Adam Ossowicki, Xiaomei Yang, Esperanza Huerta Lwanga, Francisco Dini-Andreote, Violette Geissen, Paolina Garbeva. Effects of plastic mulch film residues on wheat rhizosphere and soil properties // Journal of Hazardous Materials – 2020. Vol. 387, P.1-7. [In English]

95. Konstantinov M. M., Drozdov S. N., Yukhtin D. P. Justification of parameters of vibration tillage machines // Agricultural ENGINEERING. – Orenburg. – P.77-80. [In Russian]

96. Muratbekkyzy T., Akhmetkhanova G. A., Khazimov, K. M., Khazimov M. Zh. Device for removing polymer residues in the post-harvest period of vegetables // Digitalization of the agro-industrial complex. – Tambov, 2018. – P.51-53. [In Russian]

97. KZ 34443 B, 03.07.2020. [In Russian]

98. Kapustin. N. A. Fundamentals of the theory and calculation of machines for basic and surface tillage, sowing machines and machines for fertilization: a course of lectures // Yurginsky Institute of technology. – Tomsk: Publishing house of Tomsk Polytechnic University, 2013. – 134 p. [In Russian]

99. Turbin B. G., Lurie A. B., Grigoriev S. M., Ivanov Z. M., Melnikov S. V. Agricultural vehicles. Agricultural machinery.- L.: Publication «Mechanical Engineering», 1967. – 584 p. [In Russian]

100. Bat M. I., Dzhanelidze G. Yu., Kelzon A. S. Theoretical mechanics in examples and problems. Vol.3. – М.: Nauka, 1973. – 488 p. [In Russian]

101. Loitsyansky L. G., Lurie A. I. The course of theoretical mechanics: In 2 volumes. Vol.1. Statics and kinematics. 8-edition. revised and expanded. – М.: Nauka, Main editorial office of physical and mathematical literature, 1982. – 352 p. [In Russian]

102. Bashta T. M. Hydraulics, hydraulic machines and hydraulic drives. – М.: Engineering, 1982. – 418 p. [In Russian]

103. Lepeshkin A.V., Mikhaylin A. A., Sheypak A. A. Hydraulics and hydraulic pneumatic drive. Vol.2. hydraulic machines and hydraulic pneumatic drive: textbook. – М.: MSIU, 2003. – 352 p. [In Russian]

104. Khazimov Zh. M., Khazimov, K. M., Khazimov M. Zh. Study of field soil humidity and temperature with mechanized technology for planting vegetable seedlings using mulching film during the growing period // Bulletin of science. – Tomsk, 2018. – P.38-44. [In Russian]

105. KZ 3435 U, 30.11.2018. [In Russian]

106. Neverov A. S., Rodchenko D. A., Tsyrlin M. I. Corrosion and protection of materials: tutorial. – Minsk: Higher school, 2007. – 222 p. [In Russian]

107. Sahno O. N., Selivanov O. G., Chuklanov V. Yu. Biological stability of polymer materials / under the general editorship of Prof. T. A. Trifonova; Vladim. state. UN-t im. A. G. and N. G. Stoletovs, Vladimir, 2014. – 64 p. [In Russian]

108. Khazimov M. J., Niyazbaev A. K., Khazimov, K. M., Urymbaeva A. A., Akhanov E. L., Safargaliev A. E. Investigation of changes in the strength parameters of polymer films from atmospheric and biological factors // Research and results. – 2020. – № 3. – P.452-459. [In Russian]

109. Smyshlyaeva A. R . Correction of the thickness of polymer films during their manufacture // Polymeric material. – 2008. – № 1. – P.26-30. [In Russian]

110. Technical properties of polymer materials / V. K. Kryzhanovskii, V. V. Burlov, A. D. Panimatchenko.- SPb.: Profession, 2007. -240 p. [In Russian]

111. GOST 14236-1981 Polymer films. Tensile test method. – М.: Госстандарт, 1980. – 16 p. [In Russian]

112. GOST 11262-1980 Plastics. Tensile test method. - М.: Gosstandart, 1980. – 216 p. [In Russian]

113. Khazimov K. M., Khazimov M. Zh., Saparbayev E. T., Sultanova I. B., Zhalelov E. M. Technical features of polymer film for soil mulching // Research and results. – Almaty, 2016. – № 1. - P.271-275. [In Russian]

114. Niyazbayev A. K. Investigation of soil resistance during mulch film winding // International scientific and practical conference "VIII Global science and innovation 2020: Central Asia», A series of «Agricultural science» / Nur-Sultan (February-March 2020). - №3(3) - P.113–117. [In Russian]

115. Hoffmann F. Polymer film winding – diverse and high-quality // Polymeric material. Equipment and technologies. - 2016. – № 3. – P.46-52. [In Russian]

116. Khazimov M. Z., Khazimov K. M., Bazarbayeva T. A., Urymbayeva A. A., Bora G. C., Niyazbayev A. K. Mechanization of removal of the mulching film and flexible irriation tape from the surface of the fields // EurAsian Journal of BioSciences – 2019. – Vol. 13, P.1251-1261. [In English]

117. GOST 34393-2018 Agricultural machinery. — М.: Standardinform, 2018. – 16 p. [In Russian]

118. GOST R 15.301-2016 System development and launching of products into manufacture. Patent research. – Astana: 2005. – 29 p. [In Russian]

119. GOST R 15.301-2016 System for developing and putting products into production. Procedure for developing and putting products into production. - М.: Standardinform, 2018. - 15 p. [In Russian]

120. Effective work: AutoCAD 2004 / Sh. Ticku. - SPb.: Peter, 2004. - 1040 p. [In Russian]

121. Zharkov N. V., Finkov M. V. AutoCAD 2019. Complete guide. - Science and Technology, 2019. – 640 p. [In Russian]

122. Sokolova T. Yu. AutoCAD 2016. Two-dimensional and three-dimensional modeling. - DMK Press, 2016. - 756 p. [In Russian]

123. Harkov N. V., Finkov M. V. AutoCAD 2019. Complete guide. - Science and Technology, 2019. – 640 p. [In Russian]

124 GOST 20915 Testing of agricultural machinery. Methods for determining test conditions. – М.: Standardinform, 2015. – 23 p. [In Russian]

125. OST 10.2.2-2002 Testing of agricultural machinery. Energy assessment methods: Introduced 2003-03-03. – М.: Gosstandart of Russia, 2003. – 16 p. [In Russian]

126. OST 10.4.2-2001 Testing of agricultural machinery. Machines and tools for surface and shallow tillage. Methods for evaluating functional indicators: Introduced 2002-03-01. – М.: Gosstandart of Russia, 2002. – 14 p. [In Russian]

127. Hluzov N.Т. Some methodological approaches to the development of a complex of machines for energy-saving technologies of tillage and seeding // Achievements of science and technology in agriculture. – 2010. – № 10. – P.56. [In Russian]

128. Larin M. A., Machnev A.V., Shukov A.V. Experimental studies of a Coulter with a guide-distributor of seeds of a pneumatic seed drill // Bulletin of the Ulyanovsk state agricultural Academy. – 2012. – № 3(19). – P.118–122. [In Russian]

129. Mudrova A. A., Mudrov A. G. Energy saving issues // Bulletin of KazanSAU. – 2012. – № 1(23). – P.98–101. [In Russian]

130. Machnev A.V. Energy-saving technology and technical means of subsurface-scattered sowing of grain crops: abstract of dr. tech. sciences. – Penza, 2011. – 39 p. [In Russian]

131. STO AIST 5.3-2005. SEEDLING MACHINES. Testing of agricultural machinery. - Krasnodar, 2005. – 86 p. [In Russian]

132. Djashayev A. M. Determining the parameters of the working bodies of the vegetable seeder // Mechanization and electrification of agriculture. – 2004. – № 10. – P.29-30. [In Russian]

133. Khazimov Z.M., Bora G.C., Khazimov K.M., Khazimov M.Z., Ultanova I.B., Niyazbayev A.K. Development of a dual action planting and mulching machine for vegetable seedlings // Engineering in Agriculture, Environment and Food – 2018. – P.1-5. [In English]

134. Khazimov M. Zh., Khazimov K. M., Niyazbaev A. K. Technology and means of removing mulch film and flexible drip irrigation belts from the field surface in the post-harvest period with intensive vegetable production technology: monograph. – Almaty: NonPJSC «Kazakh national agrarian University», 2020.- 160 p. [In Russian]

135. ROCCA INDUSTRIES – Preferred Agriculture Machines [Электронный ресурс]. – URL: https://www.roccaindustries.com [In English]

136. GOST R 53056-2008 Agricultural machinery. METHODS OF ECONOMIC ASSESSMENT. - М.: Standardinform, 2018. – 24 p. [In Russian]

137. GOST 34393-2018 Interstate standard. Agricultural machinery. Methods of economic assessment. Agricultural machinery. Methods of economic evaluation. - М.: Standardinform, 2018. – 15 p. [In Russian]

138. GOST 34392-2018 Interstate standard. Agricultural machinery. Seedling machines. Test method. - М.: Standardinform, 2018. – 41 p. [In Russian]

APPENDIX A

Technical specification and work schedule

Calendar plan

| № p/p | The name of the tasks, activities aimed at the implementation of the project objectives | Duration |  | Expected results of project |
| --- | --- | --- | --- | --- |
|  |  | Beginning | Completion |  |
| 1. | To develop a technological process for removing used mulching film and flexible irrigation tapes in the post-harvest period from the field | January 2018 | June 2018 | The technological process and the constructive scheme of the device for removing the mulching film and flexible irrigation tapes will be designed |
| 1.1 | Studying the experience of the world technology achievement and technical means of removal, mulching film and flexible irrigation tapes in the post-harvest period from the field | January 2018 | March 2018 | Experience of the world technology achievement and technical means of removal, mulching film and flexible irrigation tapes in the post-harvest period from the field will be studied and report on the patent research will be prepared. |
| 1.2 | Development of technological process and constructive scheme of the device for removal of mulching film and flexible irrigation tapes | March 2018 | June 2018 | Technological process and constructive scheme of the device for removal of mulching film and flexible irrigation tapes will be designed. |
| 2. | Theoretically and experimentally substantiate the operation modes of removing the mulching film, flexible irrigation tapes and the design parameters of the device for its implementation | July 2018 | Until 1 November 2018 | Theoretically and experimentally substantiate the operation modes of removing the mulching film, flexible irrigation tapes and the design parameters of the device for its implementation will be substantiate. 1 application for Patent of RK will be applied. |
| 2.1 | Theoretical justification device design parameters for removal of mulching films and drip irrigation flexible tapes | July 2018 | September 2018 | Construction parameters and parameters of analytical dependence of designed device for removal of mulching films and drip irrigation flexible tapes will be theoretical justified. |
| 2.2 | Experimental study of the removal process of mulching film and flexible irrigation tapes | October 2018 | Until 1 November 2018 | Experimental study of the removal process of mulching film and flexible irrigation tapes will be done. Construction parameters of designed device for remove of mulching film and flexible irrigation tapes will be received. 1 (one) application for Patent of RK will be applied. |
| 3. | To carry out laboratory studies of the process of removal of the mulching film and flexible irrigation tape during combined and separate work | January 2019 | June 2019 | Laboratory studies of the process of removal of the mulching film and flexible irrigation tape during combined and separate work will be carried out. |
| 3.1 | Preparation of methods for laboratory research and equipment for realization it | January 2019 | February 2019 | Methods for laboratory research and equipment for experimental studies will be prepared. |
| 3.2 | Experimental justification of the removal mode of mulching film and flexible irrigation tape | March 2019 | June 2019 | Experimental study of the removal mode of mulching film and flexible irrigation tape will be justified. |
| 4. | To develop the design of the device for removing the used mulching film and flexible irrigation tape | July 2019 | June 2020 | Scheme, technical specifications and design documentation of the device for removing mulching film and flexible irrigation tape from the field and prototype will be developed. 1 (one) article in peer-reviewed scientific indexed journals Web of Science or Scopus and 1 (one) article in journal recommended by ССSES will be published. |
| 4.1. | Development of technical specifications and design documentation of the device for removing mulching film and flexible irrigation tape from the field | July 2019 | Until 1 November 2019 | Scheme, technical specifications and design documentation of the device for removing mulching film and flexible irrigation tape from the field will be developed. 1 (one) article in peer-reviewed scientific indexed journals Web of Science or Scopus and 1 (one) article in journal recommended by ССSES will be published. |
| 4.2. | Manufacturing an experimental device for removing mulching film and flexible irrigation tape | June 2019 | August 2019 | An experimental device for removing mulching film and flexible irrigation tape will be manufactured. |
| 5 | Test the developed experimental device for removing the mulching film and flexible irrigated tape in the field | July 2020 | September 2020 | Developed experimental device for removing the mulching film and flexible irrigated tape in the field conditions will be tested. Results (Acts and protocols of test) will be received. 1 (one) article in peer-reviewed scientific indexed journals Web of Science or Scopus and 1 (one) article in journal recommended by ССSES will be published. |
| 5.1. | Preparation for test-ing of the developed experimental device for removal of mul-ching film and flex-ible irrigation tape | July 2020 | August 2020 | Developed device for removing the mulching film and flexible irrigated tape will be prepared for field test. |
| 5.2. | Testing of an experimental device for removing a mulching film and a flexible irrigation tape in the field | September 2020 | September 2020 | Device for removing a mulching film and a flexible irrigation tape will be tested and results (protocols) of tests will be received. |
| 6. | To evaluate the technical and economic indicator of the technological process of removing the mulching film and flexible irrigation tape | October 2020 | Until 1 November 2020 | Technical and economic efficiency and recommendation and designed documentation of the device for removing the mulching film and flexible irrigation tape be received. 1 (one) article in peer-reviewed scientific indexed journals Web of Science or Scopus and 1 (one) article in journal recommended by ССSES will be published. |
| 6.1. | To evaluate the technical and economic indicator of the technological process of removing the mulching film and flexible irrigation tape | October 2020 | 1 November 2020 | Technical and economic indicators of technological process for removal of mulching film and flexible irrigation tape will be evaluate. |
| 6.2. | Development of recommendations and project documents for the industrial sample of the device for removal of mulching film and flexible irrigation tape | October 2020 | 1 November 2020 | Recommendations and design documents of device for removing the mulching film and flexible irrigated tape in the field will be developted. 1 (one) article in peer-reviewed scientific indexed journals Web of Science or Scopus and 1 (one) article in journal recommended by ССSES will be published. |

APPENDIX B Report of patent research

Патентные документы

Таблица Б.1 - Патентная документация

| № | Страна выдачи и номер охранного документа, дата публикации (охранного документа), название, классификационный индекс | Заявитель с указанием страны, номер заявки, дата подачи, конвенционный приоритет, дата публикации. | Реферат |
| --- | --- | --- | --- |
| 1 | Патент на полезную модель KZ №1782, 15.11.2016 Устройство для удаления мульчирующей пленки МПК A01G 13/02 (2006.01) | Республиканское государственное предприятие на праве хозяйственного ведения "Казахский национальный аграрный университет" Министерства сельского хозяйства Республики Казахстан Заявка 2015/0432.2 12.11.2015 | Полезная модель относится к области сельскохозяйственного машино­строения, к устройствам для снятия мульчирующей пленки из-под посевов, преимущественно овощных культур. Устройство для удаления мульчирующей пленки из - под посевов, содер­жащее раму с опорными колесами, механизм подъема пленки и барабан для наматывания мульчирующей пленки, согласно полезной модели содержит отвал для освобождения края пленки, находящегося в почве, закрепленный за отвалом механизм резки в виде барабана с качающимися ножами- лопатками, установленными по боковой поверхности барабана в шахматном порядке, и установленный под барабаном транспортер для удаленной плен­ки и скошенной растительной массы, причем над этим транспортером установлен транспортер с направляющими элементами для перемещения растительной массы, а барабан для наматывания мульчирующей пленки снабжен шипами. Разработанное устройство для удаления мульчирующей пленки обеспечивает одновременное удаление растительных отходов и может быть эффективно использовано в сельском хозяйстве при выращивании овощных куль­тур и других растений с использованием мульчирующей пленки. |
|  | Патент RU 2 302 100 C2 10.07.2007 СПОСОБ ПЕРФОРАЦИИ | Государственное научное учреждение Всероссийский научно-исследовательский институт виноградарства и | Способ касается перфорации мульчирующей рулонной полимерной пленки и относится к сельскому хозяйству, а именно для покрытия посадочных полос в школке, например, виноградных черенков. Пленку с рулона перематывают на |

Продолжение таблицы Б.1

| 2 | МУЛЬЧИРУЮЩЕЙ РУЛОННОЙ ПОЛИМЕРНОЙ ПЛЕНКИ МПК A01G 13/00 (2006.01) A01B 79/02 (2006.01) B26F 1/08 (2006.01) B26F 1/16 (2006.01) B26D 7/00 (2006.01) | виноделия им. Я.И. Потапенко (RU) Заявка 2005122059/12, 12.07.2005 | перфорационный барабан, совмещая при этом ее торцы. Одновременно с перемоткой разрезают чулок пленки с одной стороны. Затем сверлят отверстия в пленке по шаблону и вновь ведут обратную перемотку в тот же рулон. Техническим результатом является снижение затрат труда и времени, повышение качества и выхода саженцев. 1 ил. |
| --- | --- | --- | --- |
| 3 | Патент на ПМ RU 105111 U1 10.06.2011 УСТРОЙСТВО ДЛЯ УКЛАДЫВАНИЯ МУЛЬЧИРУЮЩЕЙ ПЛЕНКИ НА ГРЯДУ МПК A01B 49/00 (2006.01) | Государственное образовательное учреждение высшего профессионального образования "Петрозаводский государственный университет" (RU) Заявка 2010145090/21, 03.11.2010 | Устройство для укладывания мульчирующей пленки на гряду относится к сельскохозяйственной отрасли, а также может быть использовано для укладывания пленки для нужд других отраслей народного хозяйства. Устройство обеспечивает - уменьшение металлоемкости конструкции, удаление функций, в большинстве случаев, не востребованных потребителями. Устройство содержит центральную раму, 2 полурамы, грядообразователь, откапывающие и заделывающие диски, трамбующие ролики, соединенные с рамой при помощи шарниров. На полурамах имеется ось для крепления дополнительного рулона пленки. При этом основные рабочие органы машины выполнены в виде вращающихся дисков, которые образуют борозды для укладывания краев пленки и их заделывают |
| 4 | Патент на ИЗ SU №1530142 A1 | ЛЕВИЦКИЙ АНАТОЛИЙ САВВОВИЧ |  |

Продолжение таблицы Б.1

|  | 23.12.1989 Машина для расстила мульчирующей пленки МПК A01G 13/02 (2000.01) | ШПАН ЭМИЛЬ ЛАЗАРЕВИЧ Заявка 3823571, 10.12.1984 |  |
| --- | --- | --- | --- |
| 5 | Патент на ИЗ SU 1561836 A1 07.05.1990 Машина для укладки мульчирующей пленки МПК A01B 13/02 (2000.01) A01B 49/02 (2000.01) A01C 7/00 (2000.01) | ГУСЕВ ВЯЧЕСЛАВ МИХАЙЛОВИЧ, КОЛОБОВНИКОВА НИНА ЕГОРОВНА, ПЛАКСИНА НИНА МИХАЙЛОВНА, ВЛАСОВ ГЕННАДИЙ ГЕОРГИЕВИЧ, НИКОЛЬСКИЙ ВЯЧЕСЛАВ БОРИСОВИЧ, ШУЛЬГИН ДМИТРИЙ ВАСИЛЬЕВИЧ Заявка 4430434, 11.04.1988 |  |
| 6 | Патент на ИЗ RU №2005122 059 | Государственное научное учреждение Всероссийский научно | Способ перфорации мульчирующей рулонной полимерной пленки, включающий выполнение отверстий по шаблону, |

Продолжение таблицы Б.1

|  | 27.01.2007 СПОСОБ ПЕРФОРАЦИИ МУЛЬЧИРУЮЩЕЙ РУЛОННОЙ ПОЛИМЕРНОЙ ПЛЕНКИ МПК B29D 7/00 (2006.01) | -исследовательский институт виноградарства и виноделия им. Я.И. Потапенко ГНУ ВНИИВиВ им. Я.И. Потапенко (RU) Заявка 2005122059/12, 12.07.2005 | отличающийся тем, что предварительно пленку с рулона перематывают на перфорированный барабан большего диаметра, совмещая при этом ее торцы, одновременно с перемоткой разрезают чулок пленки с одной стороны, затем сверлят отверстия в пленке по шаблону и вновь ведут обратную перемотку в тот же рулон. |
| --- | --- | --- | --- |
| 7 | Патент SU 1319076A119870623 Устройство для полива ленты МПК G11B 5/842 (2006.01) | ПРЕДПРИЯТИЕ П/Я В-8589 SU3909886 19850611 | Изобретение относится к накоплению информации и может быть использовано для нанесения магнитных дорожек. Цель изобретения - повышение качества полива за счет уменьшения разрывов и гофр при изменении толщины ленты на склейках. Задатчик толщины полива выполнен в виде ролика 9, установленного на поливном блоке. При движении лента 13 в зоне полива прижимается поливным валиком 5 к роликам 9. Резиновые амортизаторы 15 гасят колебания поливного валика 5 при попадании склейки в зону полива и компенсируют перекос оси валика 5 при разнотолщинности ленты 13. 2 ил. 16 77 (О СО СО О О5 Ф1/е.2 |
| 8 | Патент на ИЗ KZ №26989 14.06.2013 Устройство для снятия защитной пленки МПК A01G 13/02 (2006.01) | Республиканское государственное предприятие на праве хозяйственного ведения "Костанайский государственный университет имени А. Байтурсынова" Министерства образования и науки Республики Казахстан Заявка 2012/0535.1 08.05.2012 | Изобретение относится к сельскохозяйственному машиностроению, в частности к устройствам для снятия защитной плёнки преимущественно с посевов овощных культур. Задача, которую решает изобретение, заключается в повышении качества очистки плёнки. Поставленная задача решается с помощью устройства для снятия защитной плёнки, содержащего раму с закреплёнными на ней направляющим элементом, механизмом очистки плёнки с опорной площадкой с эластичными рифлёными вальцами и ротором с контактирующими с опорной площадкой эластичными лопастями, валом со встречной |

Продолжение таблицы Б.1

|  |  |  | винтовой навивкой, механизмом наматывания плёнки, где над направляющим элементом в виде прямой трубы установлен шнек, имеющий направление вращения как у ротора и правое направление навивки витков для левой по ходу устройства его части и левое для правой части, а на раме между шнеком выше его оси и ротором крепится заслонка в виде вертикально расположенного своими плоскими поверхностями щита. |
| --- | --- | --- | --- |
| 9 | Патент на ИЗ SU 1530142A1 19891223 Машина для расстила мульчирующей пленки МПК A01G 13/02 (2006.01) | НАУЧНО-ПРОИЗВОДСТВЕННОЕ ОБЪЕДИНЕНИЕ ПО ВИНОГРАДАРСТВУ "ВИЕРУЛ" Заявка SU №3823571 19841210 | Изобретение относится к сельскому хозяйству, а именно к машинам для расстила мульчирующей пленки, например, перед посадкой черенков винограда. Цель изобретения - повышение качества подготовки посадочного места под черенок. Машина для расстила мульчирующей пленки содержит раму с опорными колесами, лемехами, барабаном для пленки, снабженным тормозом, направляющим барабаном, прижимными катками, загортачами, между которыми и за ними установлено колесо 12 с режущим рабочим органом, состоящим из двух взаимно перпендикулярно расположенных перекрещивающихся ножей 13 и 14. Нож 13 расположен в плоскости качения колеса 12 и имеет вершину, образованную двумя симметрично сходящимися лезвиями, а нож 14 - две периферийно расположенные вершины, лезвие 17 между которыми выполнено в виде симметричной впадины. При этом вершины ножей 13 и 14 расположены на одной прямой, парал-лельной образующей рабочей поверхности колеса 12.5 ил. |
| 10 | Патент SU №1773338A1 Пленкоукладчик | СРЕДНЕАЗИАТСКИЙ НАУЧНО-ИССЛЕДОВАТЕЛЬСКИЙ ИНСТИТУТ МЕХАНИЗАЦИИ И ЭЛЕКТРИФИКАЦИИ СЕЛЬСКОГО ХОЗЯЙСТВА | Изобретение относится к сельскому хозяйству, а именно к машинам для раскладки узкого пленочного мульчирующего материала над рядками высеянных семян. Цель изобретения - повышение производительности и исключение забивания почвой рабочих органов в приспособлении, содержащем раму с комплектом раскладывающих и заделывающих края пленки |

Продолжение таблицы Б.1

|  | МПК A01G 13/02 (2006.01) | Заявка SU4902298 19910114 | рабочих органов, предохранительные щитки 8 от попадания почвы нз поверхность пленки, последние выполнены по криволинейной поверхности с радиусом кривизны в диапазоне 0,7...1,5 радиуса дисковых загортачей 7 и установлены вогнутой поверхностью внутрь рядка. В результате применения изобретения повышается производительность приспособления в 1,1 раза. 3 ил. |
| --- | --- | --- | --- |
| 11 | Патент KR200485559 (Y1) Перфоратор для мульчирования пленки для сельского хозяйства МПК A01C5/04 (2006.01); A01G13/00 (2006.01); A01G13/02 (2006.01); B26F1/02 (2006.01) |  | Настоящее изобретение относится к мульчирующей пленке для мульчирующей пленки для картофеля, которая не требует сбора перфорированного винила, оставляя полностью обезвреживающую пленку во время работы с картофелем, Перфоратор. Его композиция; Разбавление; И расширение, соединенное с верхней частью перфорации и имеющее двунаправленную рукоятку на ее конце; В котором перфорированная часть имеет перфорирующую чашку, в которой режущая кромка, такая как зуб, предусмотрена только в части окружного направления в нижней части обода; В то время как перфорированная чашка выталкивается только на заданную глубину, так что пустое пространство на верхнем конце перфорированной чашечки, соответствующее 2/3 перфорированной чашечки, может служить пространством для защиты новых побегов картофеля. |
| 12 | CN107409511 (A) 2017-12-01 Ridging and film mulching dual-purpose farming machine | WEI XIANGGUANG | Изобретение относится к размалывающей и мульчирующей двухцелевой сельскохозяйственной машине. Грейферная и пленочная мульчирующая двухцелевая сельскохозяйственная машина включает в себя головку для путешествий; на дне передней бабки расположена скребковая плита для сплющивания сформированных гребней; скребковая пластина соединена с нижней частью бабки через телескопические штанги; каждый телескопический стержень включает |

Продолжение таблицы Б.1

|  | МПК A01G13/02 (2006.01) |  | фиксирующий стержень и скользящий стержень; один конецфиксирующих стержней закреплен на нижней части бабки, скользящие стержни скользя расположены в фиксирующих стержнях, а другие концы скользящих стержней вытянуты с одного конца, от передней бабки, и соединены с очищающая плита; пружины, которые прижаты к внутренним стенкам концов фиксирующих стержней, а один конец скользящих стержней расположен в фиксирующих стержнях; и нижняя поверхность, контактирующая с гребнями, скребковой пластины представляет собой дуговую поверхность, изгибающуюся вниз. Благоприятные эффекты изобретения заключаются в том, что можно предотвратить разрушение пластины скребка из-за удара; и после того, как гребни соскабливаются скребковой плитой, верхние поверхности и две стороны гребней являются плоскими. |
| --- | --- | --- | --- |
| 13 | CN107396786 (A) 2017-11-28 Mulching film laying device МПК A01G13/02 (2006.01) | CHENGDU DIANJINYUN TECH CO LTD + | Изобретение раскрывает устройство для укладки мульчирующей пленки. Устройство содержит верхнюю опору (1), в которой опорный стержень (2) расположен на конце верхней опоры (1) и соединен с тяговым валом (3); нижняя часть тяговой опоры (2) соединена с тяговой стойкой (4) для мульчирующей пленки; один конец муфтовой тяговой стойки (4) шарнирно прикреплен к тяговой опоре (2); другой конец зубчатой рейки (4) мульчирующей пленки соединен с опорным стержнем (6) для мульчирующего пленочного ролика; нижний конец опорного рулона пленки мульчирования стержня (6) жестко соединен с U-образной мульчирующей поддержкой рулона пленки (18); в мульчирующей пленочной рулонной стойке (18) расположен мульчирующий пленочный рулон (5); два конца (5) соединены с возможностью вращения с мульчирующей пленочной опорой (18); верхний конец опорного рулона пленки мульчирования |

Продолжение таблицы Б.1

|  |  |  | стержня (6) жестко соединен с прижимной ролик поперечной распорки (7); а задняя часть поперечной стойки (7) прессового валка соединена с прижимным рычагом (8). |
| --- | --- | --- | --- |
| 14 | CN107371927 (A) 2017-11-24 Film mulching machine МПК A01G13/02 (2006.01) | YANG TIANMEI + | Изобретение раскрывает пленку мульчирование машины, которая характеризуется тем, что содержит сдвигающее колесо почвы, пленку прессования опорного колеса руки, фильм, нажав колесо, опору, предельный диск, пазовое колесо, зубчатое колесо, скользящий вал, вал трансмиссии , соединитель, опорный рычаг ведущего колеса, зубчатая передача, подшипниковый узел, короткий вал, цепь, удлинительный стержень, ручка, регулировочный элемент качения с вращающимся колесом и тому подобное. Машина для мульчирования пленки имеет преимущества в том, что машина для мульчирования пленки проста в строении, с высокой практичностью и низкой стоимостью, в основном применимой к мульчированию на сельскохозяйственных угодьях, легко запускается и работает и имеет очевидную экономическую выгоду, а количественное производство может быть облегчен. |
| 15 | Патент SU №718054A1 19800229 Устройство для снятия защитной пленки МПК A01G13/02 (2006.01) | ЛЕНИНГРАДСКИЙ ЭКСПЕРИМЕНТАЛЬНЫЙ ЗАВОД СЕЛЬСКОХОЗЯЙСТВЕННОГО МАШИНОСТРОЕНИЯ | площадкой 10, которая посредством шарнира 11 может поворачиваться, что облегчает заводку пленки в устройство перед началом работы, вала 12 и П1,еточного барабана 13, перед которым закреплен опорный ролик заводку пленки в устройство перед началом работы, вала 12 и П1,еточного барабана 13, перед которым закреплен опорный ролик 14. Механизм 3 наматывания пленки состоит из ведущего вала 15 и ведомого 16 вала, который закреплен на кронштейне 17 и соединен с кронштейном 18 ведущего вала содержит цепные контуры 21-23 |
| 16 | Патент на ИЗ SU 1276299 A1 | ЦЕНТРАЛЬНЫЙ НАУЧНО-ИССЛЕДОВАТЕЛЬСКИЙ |  |

Продолжение таблицы Б.1

|  | 15.12.1986 Устройство для снятия защитной пленки МПК A01G13/02 (2006.01) | ИНСТИТУТ МЕХАНИЗАЦИИ И ЭЛЕКТРИФИКАЦИИ СЕЛЬСКОГО ХОЗЯЙСТВА НЕЧЕРНОЗЕМНОЙ ЗОНЫ СССР, БЕЛОРУССКИЙ НАУЧНО-ИССЛЕДОВАТЕЛЬСКИЙ ИНСТИТУТ КАРТОФЕЛЕВОДСТВА И ПЛОДООВОЩЕВОДСТВА Заявка 3887879, 22.04.1985 |  |
| --- | --- | --- | --- |
| 17 | Патент на ИЗ SU (11) 1 653 639 (13) A1 УСТРОЙСТВО ДЛЯ СНЯТИЯ ЗАЩИТНОЙ ПЛЕНКИ С ПОСЕВОВ ПРИ ИХ БЕСКАРКАСНОМ УКРЫТИИ МПК | МОЛДАВСКИЙ НАУЧНО-ИССЛЕДОВАТЕЛЬСКИЙ ИНСТИТУТ ОВОЩЕВОДСТВА Заявка 84616592, 06.12.1988 | Изобретение относится к сельскохозяйственному машиностроению. Цель изобретения - повышение надежности и производительности. Устройство содержит установленную на колесах 1 раму 2, на которой закреплены механизм 3 очистки пленки 4 и механизм 6 намотки. Механизм намотки состоит из фрикционной муфты 11 со следящим элементом 12 и содержит вал. на котором смонтированы ведущий и ведомый диски, поджатые пружиной. При движении устройства вал приводится в движение от ходовых колес 1 через фрикционную муфту 11. Следящий элемент 12 в |

Продолжение таблицы Б.1

|  | A01G13/02 (2006.01) |  | зависимости от диаметра бобины 18с пленкой регулирует степень поджатия дисков, находящихся в состоянии постоянного пробуксовывания, в результате чего пленка поддерживается в постоянно натянутом состоянии и плотно наматывается на бобину |
| --- | --- | --- | --- |
| 18 | Патент на ИЗ SU №1662425 A 15.07.1991 Устройство для снятия защитной пленки МПК A01G13/02 (2006.01) | БЕЛОРУССКИЙ ИНСТИТУТ МЕХАНИЗАЦИИ СЕЛЬСКОГО ХОЗЯЙСТВА Заявка: 4615073, 05.12.1988 |  |
| 19 | Патент на ИЗ SU 1739899 A1 15.06.1992 Устройство для снятия защитной пленки с временных укрытий | ХАРЬКОВСКИЙ СЕЛЬСКОХОЗЯЙСТВЕННЫЙ ИНСТИТУТ ИМ.В.В.ДОКУЧАЕВА Заявка: 4774019, 26.12.1989 | Использование: сельское хозяйство, снятие временных укрытий, включающих каркас и защитную пленку. Сущность изобретения: устройство содержит раму 1 с колесами 2, на которой смонтированы механизм 3 для намотки пленки, механизм 4 очистки пленки и механизм для снятия элементов каркаса, выполненный в виде подпружиненного кронштейна 7, установленного на нем ролика 8, прижимного механизма 12 и вибрационного механизма 14. При движении устройства вдоль укрытия ролик 8 поддевает каркас и пленку и |

Продолжение таблицы Б.1

|  | МПК A01G13/02 (2006.01) |  | выдергивает их из земли. Затем пленка очищается и наматывается на вал механизма 3, а элементы каркаса складируются на кронштейне при помощи прижимного механизма 12. |
| --- | --- | --- | --- |
| 20 | CN103749015 (A) 2014-04-30 Film-cutting removing and film-lifting winding type mulching film picking machine МПК A01G13/02 (2006.01) | WUWEI GUANGDA INDUSTRY & TRADE CO LTD CN2014139419 20140127 | Изобретение относится к области машин для сбора отходов и собирание мульчирование фильмы на ферме, в частности, к фильму резки удаления и кино-лифтинг обмотки типа мульчирование фильм комплектования машины. Фильм резки удаления и пленки подъема намоточного типа пленка мульчирующая выбора машина содержит стойку, подвеску, установленную на стойке, установленные на двух сторонах стойки и механизм передачи земельных колеса. Механизм пленка резки установлен ниже передней части стойки. Механизм подъема пленка расположена на части, в задней части механизма пленка резки, стойки. Встряхивания механизм транспортировки почвы удаления расположено на задней части механизма подъема пленки и установлены в средней части стойки. Мульчирующая пленку механизм намотки расположена над задней стойке и расположены в задней части встряхивания механизма транспортировки почвы удаления. Фильм резки удаления и кино-лифтинг обмотки типа мульчирование фильм сбор машина может завершить операции монтажера, кино, кино подъема поверхности мусора удаление, мульчирование фильма сбора и как в то время, завершить несколько рабочих процедур монтажера, Фильм атлетика, трава и солома удаления, почва пожимая-офф и как в то время, в режиме |

Продолжение таблицы Б.1

|  |  |  | комбинированной, снизить трудоемкость, сократить затраты на эксплуатацию и эффективно повысить эффективность работы. |
| --- | --- | --- | --- |
| 21 | CN103650675 (A) 2014-03-26 Rear straw smashing and plastic film residue recycling combined machine МПК A01G13/02 (2006.01) | XINJIANG KESHEN AGRICULTURAL EQUIPMENT SCI & TECH DEV CO LTD CN20131712959 20131223 | Изобретение относится к сельскохозяйственной машине, в частности к задней соломы разгрома и вторичной переработки пластмасс фильм остатков в сочетании машины, которая используется для комплексного функционирования пластиковых переработки пленочных отходов на поверхности земли сельхозугодий, разбив соломы и как. Задняя соломы разбив и переработка остаток пластиковой пленки в сочетании машина содержит стойку, устройство передачи и пешеходную колесо. Разбив устройство солома расположены в передней части стойки, край пленки напротив ходьба устройство выполнено в передней части устройства, разбив соломы и транспортировки шнек расположены на задней панели устройства, разбив соломы. Край пленки отбортовки устройство выполнено ниже транспортирующего шнека. Эластичный тип зубной пленки рейка расположена на задней стороне пленки край отбортовки устройства. Пленка отбортовки барабан расположены в задней части упругого типа зуба пленки рейка. Пленка щеткой ролика расположено на наклонной задней части пленки отбортовки барабана, пластиковая пленка остаток удаления устройства расположена в задней части пленки чистки валика А, ролик чистки ролика B расположен в положении, близком к Tai конец пластиковой пленки удаления остатка устройства А, пластиковая пленка остаток удаления устройство В расположены в задней части валика чистки валика B, устройство удаления дуги расположен под пластиковой пленки удаления остатка устройства В, и пленка |

Продолжение таблицы Б.1

|  |  |  | щеткой ролика С размещено в задней части пластиковой устройства удаления пленки остаток В. заднего соломы разбив и переработка остаток пластиковой пленки в сочетании машину разумно в структуре и высоким качеством работы, и может эффективно перерабатывать пластиковые отходы пленки в процессе возвращения соломы на поля и эффективно улучшить эффективность работы. |
| --- | --- | --- | --- |
| 22 | CN201986357 (U) 2011-09-28 Drum-type mulching film picking up machine МПК A01B43/00 (2006.01) | QINGYANG QIANJIN MACHINE MFG CO LTD CN2011284583U 20110328 | Полезная модель относится к барабанного типа для мульчирования пленкой поднимая машину, которая включает стойку, фильм сбор лопату, коробку фильм сбора и механизм передачи, в котором солома вспашку лопатой, расположенный на нижнем конце средней траверсе стойки Пленка выбора лопатой расположено на нижнем конце задней поперечине стойки вращающийся барабан расположен между средней поперечной балки и задней поперечной балки стойки коробка пленка сбора расположено на стойке за вращающегося барабана для механизма передачи, ведущие колеса расположены соответственно на обеих сторонах стеллажа, в котором внутренняя сторона рабочего колеса на одной стороне стойки соосно соединен с ведущей звездочке, которая соединена с конусной шкива, Привод находится расположены на внутренней стороне конуса шкива, ведомая шестерня находится в зацеплении и соединен с ведущей шестерней, и один конец ведомой шестерни соединен с вала барабана. |
| 23 | CN203423918 (U) 2014-02-12 Clearing device for large | SANMENXIA CENTURY AGRICULTURAL MACHINERY ASSEMBLY CO LTD | Полезную модель раскрывает очистное устройство для большого мульчирования пленкой. Очистное устройство содержит устройство сгребание собранный на раме типа колеса транспортного средства минутах через опорную раму в режиме подъема, Боковое |

Продолжение таблицы Б.1

|  | mulching film МПК A01B43/00 (2006.01); A01G13/02 (2006.01) | CN20132371803U 20130625 | устройство снабжено столбчатой тела собранном на опорной раме в режиме вращающегося, приводной механизм вождения столбчатые тела, чтобы повернуть неподвижно расположены на опорной раме, симметрично расположенных собрав-зубы, простирающиеся наружу в верхнем ряду и нижнем ряду расположены на периферийной поверхности цилиндрического тела в диаметральном направлении, корзина мульчирующая пленка крепится к опорной рамы, корзина мульчирующая пленка неподвижно закреплены с собирающей рейк над Загребая устройства, сбор рейка используется для обстреливая мульчи пленку на Загребая устройства в корзине мульчирующая пленки и снабжен телескопической части стержня и головки гвоздя который прикреплен к части телескопической штанги и используются для подключения к мульчи фильм. |
| --- | --- | --- | --- |
| 24 | CN203482590 (U) 2014-03-19 Straight tooth type roller mulching film recovery device МПК A01B43/00 (2006.01); A01G13/02 (2006.01) | DINGXI SANNIU AGRICULTURAL MACHINERY MFG CO LTD CN20132593889U 20130925 | Полезная модель относится к мульчирования пленкой очистки сельскохозяйственных механические инструменты, в частности, прямой тип зубов ролик мульчирование восстановления Фильм устройства. Прямой тип зубов ролик мульчирование восстановления фильм устройство сочетает в себе сбор пленки и очистку в целом, и разумно в структуре, легко и удобно использовать, с низким потреблением энергии и высокой эффективности. |
| 25 | CN203675553 (U) 2014-07-02 Film cutting, removing, lifting and winding type mulching film pick-up machine | WIWEI GUANGDA INDUSTRY AND TRADE CO LTD + CN2014252660U 20140127 | Полезную модель раскрывает монтажера, удаление подъема и обмотки типа мульчирование фильм пикап машину и относится к технической области сельхозугодий отходов мульчирования сбора пленки и пикап машин. Мульчирующая пленка пикап машина содержит корпус машины, суспензии раму, расположенную на раме машины, земельные колеса расположены на двух сторонах рамы станка и механизма передачи, механизм пленка резки расположен |

Продолжение таблицы Б.1

|  | МПК A01B43/00 (2006.01); A01G13/02 (2006.01) |  | под передней части машины рамы, механизм подъема пленка расположена в положении, за механизмом пленка резки на раме машины, встряхивая почвы и удаление транспортирующий механизм выполнен за механизма подъема пленки, почва встряхивания и удаления транспортирующий механизм выполнен в середине из рамы машины, мульчирующая намотанной пленки механизм расположен над задней части рамы машины, а мульчирующая намотанной пленки механизм устроен позади почва встряхивания и удаления транспортирующего механизма. Машина может закончить фильм резки, пленки атлетика, поверхность пленки для удаления примесей, мульчирование пленки сбор и как на один раз, сочетание работы из множества рабочих процедур, таких как пленки резки, кино подъема, сорняков и соломы удаления и почвы пожимая может будет завершена в то время, интенсивность труда снижается, работает стоимость сохраняется, и эффективность работы эффективно улучшается. |
| --- | --- | --- | --- |
| 26 | CN203859999 (U) 2014-10-08 Mulching film clearing machine МПК A01B43/00 (2006.01); A01G13/02 (2006.01) | LI WENZE CN20142229832U 20140429 | Мульчирование фильм очистка машина содержит подачи почвы колесо, вибрационный экран, рамку пучок, защитную пластину, пропускаемого ведро, универсальные диски, посадки на мель клетку, вентилятор, подавая трубу, вал передачи, коробка передач, рычаг подвески и трактор. Submit CorrectionsClose Подача почвы колесо расположено на передней части рамы пучка, вибросито, расположена на задней стороне подачи колеса почвы, нижний конец задней стороне вибрационного сита снабжена переплетения клетке и вентилятора две группы универсальных колес расположены на левом нижнем конце и нижнем правом конце середине кадра пучка, подача трубы расположены на наружном конце воздушного порта вентилятора, и верхний порт подачи Труба приводит к |

Продолжение таблицы Б.1

|  |  |  | пропуска ведро расположенных на верхнем конце перегородки. Защита пластина содержит пылезащитные чехлы, расположенные с двух сторон. Пылезащитной крышки расположены на верхних концах и на левой стороне и на правой стороне подачи грунта колеса и вибрационного сита и неподвижно установлен на раме пучка, рычаг подвески неподвижно установлен на переднем конце рамы пучка и связана с трактором, и устройство передачи приводит в движение колесо подачи масла, вибросито, переплетения клетку и вентилятор. |
| --- | --- | --- | --- |
| 27 | CN203984914 (U) ― 2014-12-10 Mulch film recovery device МПК A01B43/00 (2006.01); A01G13/02 (2006.01) | XIA ZHENGFU | Полезная модель относится к сельскохозяйственной машины, в частности устройства мульча восстановления фильм. Устройство содержит стойку 1, прокатный ствол 2, щетка 3, батарею 4, электродвигатель 5, а кнопочный выключатель 6, и колесо 12. Устройство характеризуется тем, что при прокатке ствола 2 подвижно соединена со стойкой 1 через I10 подшипника сиденья с подшипником и II11 подшипника сиденья с подшипником и сбоку подвижного ствола 2 снабжена щеткой 3. Щетка 3 может перемещаться в небольшом диапазоне от стеллажа 1, тем самым регулируя расстояние между щеткой 3 и подвижного ствола 2. Когда устройство перемещается вперед, вращение электродвигателя 5 можно регулировать с помощью кнопки переключателя 6, и подвижного ствола 2 приводится через ременную передачу на rotat3. Когда подвижного ствола 2 вращается по часовой стрелке, мульча пленка на землю может быть ранеными на подвижного ствола 2 и 3 щетки можно очистить почву на мульчи пленки в процессе намотки. |

Продолжение таблицы Б.1

|  |  |  | Устройство может очистить мульчи фильм на земле быстро и удобно. |
| --- | --- | --- | --- |
| 28 | C N204047053 (U) ― 2014-12-31 Digging-tooth film recycling machine МПК A01B43/00 (2006.01); | BORTALA MONGOL AUTONOMOUS PREFECTURE YONGQI WATER SAVING EQUIPMENT SCIENCE & TECHNOLOGY DEV CO LTD; LI JING | Полезную модель раскрыта рытье зуба переработки пленки автомат, содержащий суспензию, основную раму и механизм передачи. Зубья долота (11) расположены на нижней части основной рамы. Сбор устройство выполнено по землеройной зубов (11) и включает в себя привод колесной пары (22) и цепи (23) расположены на ведущее колесо набора (22). Поперечные стержни (24) и доставка зубов (21), выстроенные расположены на цепях (23) Экран (25) параллельно расположены на цепях (23) и поперечных стержней (24). Щеточный валик (32) с равномерно распределенными щетинок (31) и коллектор расположены в задней части захватного устройства. По сравнению с уровнем техники, рытье зуб утилизацию машина фильм лучше в силу утилизации, проще по конструкции и выше в выборе эффективность, лучше подобрать кусочки пленки различных размеров и особенно может выкопать, ясно и перерабатывать фильм остается похоронен в почва. |
| 29 | US 5,386,876 MACHINE FOR REMOVING WIDE STRIPS LAD OUT ON THE GROUND A01D 33/08 (1995.02) | Guy Lavo, Sancheville | Машина 1 содержит раму 4, которая состоит из двух продольных балок 5, отделенных друг от друга, и одной передней поперечной балки 6, которая соединяет их и на которой предусмотрено средство для крепления рамы к трактору 7. В задней части продольные балки 5 посредством рычагов 5а, проходящих вниз, имеются два опорных колеса 8. В своей задней части продольная балка 5 имеет два больших |
| Продолжение таблицы Б.1 |  |  |  |
|  |  |  | коаксиальных колеса 9 со спицами, которые не касаются земли и связанны во вращении с поперечным подвижным валом 10. Колеса 9 могут приводиться во вращение с помощью ходовых колес 11, которые прижимаются к их периферии сверху и которые установлены на общем поперечном валу 12, приводимом в движение гидравлическим двигателем, при этом колеса 11 и вал 12 установлены на конец двух противоположных рычагов 14, шарнирно соединенных с продольными балками 5 и на которые воздействуют пружины или натяжители 15, действующие в направлении, которое прижимает ходовые колеса 11 к колесам 9 для получения фрикционного привода, который можно регулировать натяжением пружин или натяжителей 15. В своей передней части рама 4 с помощью рычагов 5b, идущих вниз, опирается на две расцепленные части отвала 16, которые направлены вперед и находятся на уровне земли, причем расстояние между отвалами 16 немного меньше ширины мульчирующей пленки 2. Расстояние между отвалами 16 регулируется для того, чтобы можно было приспосабливаться к разной ширине удаляемых пленок. Перед колесами 9 и за отвалами 16 продольные балки 5 несут свободные для вращения концы поперечного вала 17, между которыми расположен поперечный барабан 18. Поперечный барабан 18 имеет на своих концах две противолежащие друг другу радиальные пластины 19, и его периферийная часть, образована восемью поперечными стержнями 18а цилиндрического поперечного сечения, которые находятся в их средней части, которые простираются прямолинейно до пластин 19 и которые равномерно распределены таким образом, чтобы образовывать две противоположные конусные секции, барабан 18 имеет соответственно диаметры, которые уменьшаются по направлению к каждому из его концов. |
| Продолжение таблицы Б.1 |  |  |  |
|  |  |  | Кроме того, рама 4 имеет две цилиндрические щетки 20 для отчистки мусора находящегося на поверхности пленки, которые связаны с двумя конусообразными частями барабана 18 и расположены так, что эти щетки 20 вращаются приводясь во вращение гидравлическим двигателем 21. Кроме того, они могут быть отрегулированы по высоте относительно барабана 18. Две цилиндрические щетки 20, которые расположены по обе стороны от средней радиальной плоскости барабана 18 и которые простираются по всей его длине, наклонены относительно продольного направления полосы таким образом, что их смежные концы находятся позади их другие концы, причем их смежные концы опираются на поперечную балку 22 рамы 4. Сзади и ниже двух щеток 20 предусмотрены два дефлектора 23, которые наклонены назад и которые имеют перевернутый задний край 24, при этом дефлекторы 23 наклонены к задней части и к боковым сторонам машины. Между колесами 9 и барабаном 18 предусмотрен поперечный ролик 25, концы которого опираются на два в зацепленных рычагах 26, другие концы которых шарнирно соединены с продольными балками 5. Один из его рычагов 26 соединен стяжным стержнем 27. Подвижный элемент для регулировки гидравлического распределителя 28 установлен на гидравлическом контуре двигателя для привода ходовых колес 11. |

Выводы о выполнении регламента поиска

Регламент поиска по теме: «Технология и средства удаления мульчирующей пленки и гибких поливных лент капельного орошения с поля в после уборочный период»

Всего отобрано по патентной документации: 29

Из них по странам: Казахстан – 2, Россия – 13, Китай – 12, Республика Корея – 1.

При патентно-информационном поиске найден наиболее близкий аналог KZ №1782 от 15.11.2016 г. «Устройство для удаления мульчирующей пленки», совпадающим признаком являются то, что устройство для удаления мульчирующей пленки, содержащее раму с опорными колесами, механизм подъема пленки и барабан для наматывания мульчирующей пленки, также отвал для освобождения края пленки, находящегося в почве, закрепленный за отвалом механизм резки в виде барабана с качающимися ножами лопатками, установленными по боковой поверхности барабана в шахматном порядке, и установленный под барабаном транспорт для удаленной пленки и скошенной растительной массы, причем над этим транспортером установлен транспортер с направляющими элементами для перемещения растительной массы, а барабан для наматывания мульчирующей пленки снабжен шипами, отличием является то, что барабан для наматывания мульчирующей пленки выполнен в виде принудительного привода от гидромотора и без шипов.

APPENDIX W

List of patents and applications

APPENDIX G

Results of calculations of the theoretical part

Г.1 Расчет прочностных характеристик трубы из ПВХ

Под воздействием наматываемой  пленки  в трубе возникают касательные напряжения, которые стремятся деформировать ее. Касательные напряжения возникают в результате натяжения  наматываемой пленки за счет сопротивления  почвы при вытаскивании пленки из почвы. Расчет прочности трубы ПВХ производился с использованием программного обеспечения Inventor Professional 2018, которое определяет устойчивость трубы при соответствующих касательных напряжениях: при максимальном сопротивлений почвы; при разрыве пленки.

Выбранная модель трубы  с фиксаторами на торцевой части представлена на рисунке Г.1.

Рисунок Г.1 – Модель спаренного устройства

Для расчета  напряжений возникающих в трубе исходные характеристики трубы из ПВХ представлены в таблице 1.

Таблица Г.1 – Характеристики трубы из ПВХ для расчета напряжений

| Массовая плотность | 1,4 г/см3 |
| --- | --- |
| Предел текучести | 46,53 MПа |
| Окончательный предел прочности растяжения | 52,36 MПа |
| Модуль Юнга | 3,4 ГПа |
| Коэффициент Пуассона | 0,4 |
| Модуль упругости при сдвиге | 1,21429 ГПа |
| Диаметр трубы | 50 мм |

Г.2 Результаты расчета напряжений трубы из ПВХ

В результате расчета напряжений трубы из ПВХ с учетом  прочностных показателей полимерных пленок от различных факторов и сопротивления почвы при намотке получены результаты расчета представленные на рисунках Г.2 и Г.3.

Рисунок Г.2 – Смещения трубы при максимальном

натяжении пленки перед разрывом

Рисунок Г.3 – Коэффициент запаса прочности устройства

Как видно из графика в результате воздействия сопротивления почвы при намотке мульчирующей пленки смещение трубы от центральной ее оси (в условиях без прогиба) составляет 451 мм. Данное смещение принято допустимым для сохранения устойчивого состояния трубы между прижимными конусами.

APPENDIX D

Data of experimental study

Д.1 Методика определения углов и коэффициентов трения отходов растительного происхождения

Углы трения по стальной поверхности без давления определялись в покое и в движении на приборе, схема которого представлена на рисунке Д.1.

1 – стальной лист; 2 – станина горизонтальная; 3 – вертикальная опора;

4 – конечный блок; 5 – промежуточный блок; 6 – секторный механизм;

7 – нить для привода стального листа

Рисунок Д.1 – Схема и общий вид прибора для определения углов естественного откоса и трения по стальной поверхности

На поверхности стального листа 1, находящегося в горизонтальном положении, свободно размещался исследуемый материал произвольной массы. Затем лист вручную поднимался лебедкой 4 до тех пор, пока исследуемый материал не приходил в движение. Положение листа фиксировалось, и замерялся угол его наклона, с помощью секторного механизма 6. При определении углов трения в движении лист 1 поднимается на фиксированный угол и на него размещается определенная порция исследуемого материала. По мере увеличения угла наклона листа1, фиксировалось положение, при котором исследуемый материал, начинал сползать без задержки по поверхности трения. Для достоверной оценки углов трения опыты проводились в пятикратной повторности. Температура и влажность исследуемого материала во всех опытах поддерживалась в пределах: температура 298 +1К, влажность 10+2%.

Коэффициент трения при низких давлениях (в пределах 500…1500 Па) определялась на установке, схема которой показана на рисунке Д.1.

Для определения коэффициента трения зеленой массы от давления использовалось устройство, разработанное сотрудниками.

Схема установки для определения внешнего трения (коэффициента, трения скольжения) показана на рисунке Д.1.

Методика определения динамического коэффициента трения заключается в следующем. Исследуемый материал засыпается в короб 7 разравнивается. На материал накладывается пластина, давление от которой равно давлению на кожухе распределителя трудносыпучих материалов или на витках шнекового питателя пресса, создаваемого при перемещении материала за счет центробежных сил.

1-рама; 2- опорный наконечник кронштейна; 3- тензометрический датчик;

4- кронштейн; 5- вал цилиндрического барабана; 6- цилиндрический барабан трения; 7- короб; 8- приводной вал; 9- ось промежуточного колеса;

10- промежуточное колесо; 11- колесо на валу электродвигателя;

12- электродвигатель; 13- крышка короба; 14- ручка крышки; 15- грузик;

16- исследуемый материал; 17- опора промежуточного колеса; 18-опора цилиндрического барабана; 19- осциллограф;

20- частотный преобразователь; 21- сменный шкив

Рисунок Д.2 – Схема и общий вид установки для определения коэффициентов трения

Барабан 6 приводится во вращение от мотор-редуктора 12. При работе прибора шарнирно укрепленный на оси короб  7, за счет сил сцепления  между поверхностью  «дорожки» и исследуемым материалом, стремится повернуться в направлении вращения барабана 6, оказывая через опорные пластины 2 давление на кронштейн 4 равного сопротивления с датчиками.

По показанию милливольтметра по тарировочному графику определялось истинное значение усилий на перемещение материалов растительного происхождения.

Коэффициенты трения рассчитывались по формулам:

- для статистических коэффициентов трения –

,                                                (Д.1)

- для динамических коэффициентов трения –

,                                              (Д.2)

гдеfc и fд – соответственно статический и динамический коэффициент трения;

Fc и Fд – сила трения растительного продукта в начальный момент сдвига коробки и при установившемся движении, Н;

Рс и Рд – сила нормального давления, Н.

Вся серия опытов проводилась в 3-кратной поверхности при изменяемых значениях нормального давления, скорости перемещения, температуры и влажности исследуемого материала.

Д.2 Определение изменения прочностных показателей полимерных пленок от различных факторов

Полимеры, как и многие другие материалы, способны разрушаться под воздействием факторов внешней среды. Однако в отличие от металлов и бетона термин «коррозия» к ним неприменим, а используется термин «деструкция». Устойчивость полимерных материалов к воздействию факторов внешней среды принято характеризовать атмосферостойкостью. Атмосферостойкость – это способность полимерных материалов выдерживать воздействие различных атмосферных факторов (солнечная радиация, тепло, кислород воздуха, влага, промышленные выхлопные газы и др.) в течение продолжительного времени без значительного изменения внешнего вида, а также эксплуатационных свойств (физико-механических, диэлектрических и др.). Также на полимеры воздействуют живые организмы, вызывающие биоповреждения. Биоповреждение (биологическое повреждение) — это любое изменение (нарушение) структурных и функциональных характеристик объекта, вызываемое биологическим фактором. При этом биоповреждение материалов микроорганизмами происходит с участием не одной какой–либо группы, а с участием всех существующих видов микроорганизмов.

Атмосферостойкость полимерных пленок определялись в естественных условиях. При испытаниях образцы закрепили на поверхности почвы, расположенных на открытой площадке (рисунок Д.3).

Рисунок Д.3 – Заложенные образцы в полевых условиях

Испытание полимерных пленок проводилось согласно требованиям ГОСТ 14236-81. Для испытания полимерных пленок применялись образцы в форме прямоугольника шириной 25 мм, длиной не менее 150 мм (рисунок Д.4).

Рисунок Д.4 – Размеры образцов разных видов полиэтиленовой пленки

Числовые значения размеров пленки приведены в таблице Д.1.

Таблица Д.1 – Числовые значения образцов испытания

| Параметры | Размеры, мм |
| --- | --- |
| Общая длина l1, не менее | 250 |
| Расстояние между метками, определяющими положение кромок зажимов на образце l2 | 170 ±5 |
| Расчетная длина l0 | 50 ±1 |
| Ширина головки b1 | 25 ±0,5 |
| Толщина d | 0,03 0,05 0,08 0,1 0,1 (черная) 0,12 (черная) |

Перед испытанием на центральную часть образца наносились метки, ограничивающую расчетную длину l0, длина которой составляла 50 мм (без повреждения образцов). Толщину и ширину образцов измеряли в трех местах, в середине образца и на расстоянии 5 мм от краев меток прибором ЕТ – 11 Р, который перед проведением исследования предварительно калибровался с помощью стандартной пленки (рисунок Д.5). Из полученных значений вычислялись средние арифметические, по которым вычисляют начальное поперечное сечение А0.

Рисунок Д.5 – Прибор для замера толщины пленки с калибровочной пленкой

Образцы закреплялись в зажимы испытательной машины (рисунок Д.6) по меткам, определяющим положение кромок зажимов, таким образом, чтобы продольные оси зажимов и ось образца совпадали между собой и с направлением движения подвижного зажима. Зажимы затягивались равномерно, чтобы не было проскальзывания образца в процессе испытания, но при этом не происходило его разрушение в месте закрепления. Далее настраивался прибор для замера деформаций (при этом учитывалось, что l0 – расчетная длина, мм).

1 – рукоятка; 2 – опора, 3 – мерная шкала, 4 – образец растяжения;

5 – тензодатчик

Рисунок Д.6 – Схема и общий вид испытательного прибора на растяжение

Обработка результатов на прочность (σ) в МПа (Н/мм2) проводилась по следующим формулам:

прочность при растяжении (σz)

| σz=FmaxA0 | (Д.3) |
| --- | --- |

прочность при разрыве (σr)

| σr=FrA0 | (Д.4) |
| --- | --- |

где Fmax – максимальная растягивающая нагрузка при испытании на растяжение, Н;

Fr – растягивающая нагрузка в момент разрыва, Н;

A0 – начальное поперечное сечение образца, мм2.

Относительное удлинение (ε) в процентах вычислялось по формулам:

относительное удлинение при максимальной нагрузке (εz)

| εz=Δl0zl0∙100, | (Д.5) |
| --- | --- |

относительное удлинение при разрыве (εr)

| εr=Δl0rl0∙100 | (Д.6) |
| --- | --- |

где l0 – начальная расчетная длина образца, мм;

l0z – изменение расчетной длины образца в момент достижения максимальной нагрузки, мм;

l0r – изменение расчетной длины образца в момент разрыва, мм;

За результат испытания принимались среднее арифметические пяти определений, вычисленное до второй значащей цифры.

Д.3 Методика определения сопротивления почвы при намотке

Для определения сопротивления почвы при намотке использовалась установка для намотки мульчирующей пленки (рисунок Д.7), которая состоит из рамы-стойки 1, вала с намоточной гильзой 2 и динамометрического ключа 3.

1 – рама-стойка; 2 – вал с намоточной гильзой, 3 – динамометрический ключ, 4 – мульчирующая пленка; 5 – корневые остатки

со срезанными стеблями

Рисунок Д.7 – Схема и общий вид установки для определения сопротивления почвы при намотке

Д.4 Изменение прочностных показателей полимерных пленок от различных факторов

По мере повышения температуры окружающей среды на образцах из-за окислительных процессов, улетучивании пластификаторов, противоустари-телей и т.д., привело к изменению исходных свойств, в частности к росту жесткости и хрупкости. Также на образцах с истечением времени произошло изменение цвета, появились трещины (рисунок Д.8).

Рисунок Д.8 – Общий вид и полевые экспозиции образцов разных видов полиэтиленовой пленки

Результаты исследования на прочность при растяжении, прочности при разрыве, относительного удлинения при максимальной нагрузке и относительного удлинения при разрыве получены графические интерпретации в виде зависимостей на рисунках Д.9, Д.10, Д.11, Д.12 значение которых приведены в таблице Д.2.

Рисунок Д.9 – Сравнительные показатели полиэтиленовых пленок на прочность при растяжении с различной толщиной

Рисунок Д.10 – Сравнительные показатели полиэтиленовых пленок на прочность при разрыве с различной толщиной

Рисунок Д.11 – Сравнительные показатели полиэтиленовых пленок на относительное удлинение при максимальной нагрузке с различной толщиной

Рисунок Д.12 – Сравнительные показатели полиэтиленовых пленок на относительное удлинение при разрыве с различной толщиной

Таблица Д.2 – Результаты испытания определения на прочность и относительного удлинения

| № | Толщина d, мкм | Продолжительность τ, дни |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 15 | 30 | 45 | 60 | 75 | 90 | 105 |
| Прочность при растяжении σz, МПа |  |  |  |  |  |  |  |  |  |
| 1 | 30 | 9,42 | 9,38 | 8,98 | 9,29 | 13,28 | 11,81 | 10,18 | 9,5 |
| 2 | 50 | 8,74 | 8,21 | 8,2 | 8,01 | 8,75 | 10,61 | 9,92 | 8,97 |
| 3 | 80 | 9,8 | 9,73 | 9,44 | 9,73 | 10,196 | 10,4 | 12,72 | 11,7 |
| 4 | 100 | 11,65 | 11,83 | 11,92 | 12,36 | 10,48 | 9,17 | 10,81 | 9,83 |
| 5 | 100 (черная) | 13,22 | 13,28 | 11,11 | 10,55 | 10,73 | 12,54 | 11,84 | 11,26 |
| 6 | 120 (черная) | 13,4 | 14,04 | 11,8 | 10,49 | 10,94 | 11,97 | 9,36 | 9,3 |
| Прочность при разрыве σr, МПа |  |  |  |  |  |  |  |  |  |
| 1 | 30 | 9,42 | 9,36 | 9,31 | 7,45 | 10,98 | 5,85 | 5,18 | 4,97 |
| 2 | 50 | 8,59 | 7,94 | 7,76 | 7,66 | 8,36 | 7,32 | 7,91 | 7,9 |
| 3 | 80 | 9,52 | 9,39 | 9,21 | 9,55 | 8,26 | 8,18 | 8,42 | 8,22 |
| 4 | 100 | 11,06 | 11,34 | 11,28 | 12,27 | 9,14 | 8,38 | 8,45 | 8,2 |
| 5 | 100 (черная) | 11,94 | 10,73 | 9,66 | 9,5 | 8,9 | 8,72 | 8,84 | 8,15 |
| 6 | 120 (черная) | 13,27 | 13,66 | 11,04 | 9,4 | 9,05 | 9,62 | 6,4 | 8,22 |
| Относительное удлинение при максимальной нагрузке εz, % |  |  |  |  |  |  |  |  |  |
| 1 | 30 | 444 | 366 | 342 | 144 | 110 | 132 | 110 | 118 |
| 2 | 50 | 352 | 216 | 200 | 172 | 110 | 116 | 116 | 114 |
| 3 | 80 | 404 | 364 | 356 | 346 | 160 | 180 | 140 | 132 |
| 4 | 100 | 434 | 360 | 396 | 494 | 198 | 152 | 122 | 120 |
| 5 | 100 (черная) | 414 | 310 | 216 | 118 | 110 | 158 | 116 | 106 |
| 6 | 120 (черная) | 696 | 508 | 342 | 118 | 110 | 124 | 112 | 124 |
| Относительное удлинение при разрыве εr, % |  |  |  |  |  |  |  |  |  |
| 1 | 30 | 444 | 390 | 358 | 222 | 130 | 180 | 110 | 188 |
| 2 | 50 | 276 | 244 | 230 | 202 | 116 | 156 | 134 | 146 |
| 3 | 80 | 416 | 396 | 380 | 392 | 210 | 236 | 202 | 162 |
| 4 | 100 | 352 | 382 | 426 | 598 | 292 | 184 | 178 | 150 |
| 5 | 100 (черная) | 424 | 344 | 250 | 150 | 122 | 192 | 126 | 154 |
| 6 | 120 (черная) | 704 | 538 | 416 | 148 | 122 | 150 | 122 | 120 |

APPENDIX E

Terms of reference for the development

PPENDIX Zh

Design documentation of an  experimental model

APPENDIX I

General view of an experimental model of a machine for removing mulch film and flexible drip tape

APPENDIX K

Energy performance of machine for

remove mulch film and flexible drip tape

Для определения твердости почвы был использован пенетрометр ПСГ-МГ 4. Исследования проводились непосредственно в полевых условиях, где должен был пройти проверку разрабатываемая экспериментальная машина для удаления мульчирующей пленки и гибкие капельные ленты (рисунок К.1).

Рисунок К.1 – Грунтовый пенетрометр в состоянии проверки почвы

Влажность определялась общепринятым методом высушивания. Для этого брались пробы на каждом участке и различных глубинах 0…5 см, 5…10, 10…15 см в специальные стаканчики (бюксы). Взятые пробы высушивались в сушильном шкафу при t=1050С в течение 6 часов. После высушивания влажность определялась в процентах по формуле:

| , % | (К.1) |
| --- | --- |

где ,- масса бюксы с почвой соответственно до и после высушивания, г, - масса бюксы без почвы, г.

Затем определялась средняя влажность по глубине:

| , % | (К.2) |
| --- | --- |

где  - число замеров по глубине,

- влажность в процентах на заданной глубине.

Таблица К.1 – Влажность и твердость почвы на участке заделанного края пленки

| Участок | Глубина замера, см | Влажность почвы, % | Твердость, МПа |
| --- | --- | --- | --- |
| Заделанный край пленки | 5 | 43,7 | 2,8 |
|  | 10 | 46,4 | 2,93 |
|  | 15 | 51,4 | 3,14 |

В процессе проведения экспериментальных исследований в полевых условиях были произведены замеры мощности, потребной на холостой ход и рабочие ходы для различных вариантов установки рабочих органов. Для этого регистрировались специальной аппаратурой давление рабочей жидкости на входе и выходе гидромотора.

Мощности, потребляемой на привод рабочих органов испытуемого агрегата с гидравлическим приводом рабочих органов N, определяется выражением

| N=Qт∙∆P61,2∙ηгм, кВт | (К.3) |
| --- | --- |

где, Qт – теоретическая подача насоса, л/мин;

ΔP – перепад давления в гидромоторе, мПа;

ηгм – гидромеханический КПД гидромотора.

Программируемое реле ПР200-24.4.2.0 применялась для решения задач автоматизированного подсчета перепада давления в гидромоторе. Для автоматического контроля перепада давления на входе гидромотора был установлен датчик давления ПД100-ДИ40.0-111-0.5 и на выходе датчик давления ПД100-ДИ25.0-111-0.5 с точностью измерения ±0,5% ВПИ фирмы «Овен». При проведении полевых испытании также измерялись частота оборотов гидромотора, счетчиком оборотов (тахометром) ТХ01-224.Щ2.Р.RS, скорость движения агрегата при помощи программируемого реле ПР200-24.4.2.0. Для этого применялись индуктивные бесконтактные выключатели ISB A4A-31P-5-LZ (рисунок К.2).

Рисунок К.2 – Установка датчиков на экспериментальной машине:

а) для регистрации перепада давления в гидромоторе; б) для регистрации оборотов вала гидромотора; в) для регистрации оборотов колеса машины

с) блок-регистратор

Программирование реле ПР200-24.4.2.0 производилось написанием алгоритма на языке FBD (МЭК 16131:3) с помощью среды программирования OWEN Logic (рисунок К.3).

Рисунок К.3 – Интерфейс среды программирования OWEN Logic для создания алгоритмов работы программируемого реле

Для визуализации и управления измерительными процессами программируемое реле ПР200-24.4.2.0 было подключено к панели оператора ETG 7˝ серии ONI с оперативной и постоянной памятью для обработки и сохранения информационных данных. Программное обеспечение для создания пользовательского приложения на панели оператора производилось с помощью программы ONI Visual Studio (рисунок К.4). Где можно было задать параметры радиуса качения колеса машины, коэффициента учитывающего проскальзывание колеса машины, теоретическую подачу насоса и гидромеханический КПД гидромотора.

Рисунок К.4 – Интерфейс программного обеспечения ONI Visual Studio при создании алгоритма работы и графической панели оператора

Перед выполнением технологических операций механизмы машины устанавливались на оптимальные режимы применительно к условиям испытаний и техническим требованиям к удалению мульчирующей пленки и гибких капельных лент. Испытания проводились на различных скоростях агрегата (рисунок К.5)

Рисунок К.5 – Испытание экспериментальной машины для удаления мульчирующей пленки и гибких капельных лент

Энергия, передаваемая приводным механизмом машины для удаления мульчирующей пленки и гибкой капельной ленты, расходуется в основном на поднятие мульчирующей пленки с поверхности поля, намотку мульчирующей пленки и капельной ленты. В результате обработки данных, записанных во время  полевых испытаний, построены графики зависимости расходуемой мощности от оборотов гидромотора и поступательной скорости агрегата (рисунки К.6 – К.18).

Рисунок – К.6 Изменение потребляемой мощности в зависимости от перепада давления при средней скорости агрегата Vср = 0,52 м/с

Рисунок – К.7 Изменение потребляемой мощности в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 0,52 м/с

Рисунок – К.8 Изменение перепада давления в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 0,52 м/с

Рисунок – К.9 Изменение потребляемой мощности в зависимости от перепада давления при средней скорости агрегата Vср = 0,96 м/с

Рисунок – К.10 Изменение потребляемой мощности в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 0,96 м/с

Рисунок – К.11 Изменение перепада давления в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 0,96 м/с

Рисунок – К.12 Изменение потребляемой мощности в зависимости от перепада давления при средней скорости агрегата Vср = 1,58 м/с

Рисунок – К.13 Изменение потребляемой мощности в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 1,58 м/с

Рисунок – К.14 Изменение перепада давления в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 1,58 м/с

Рисунок – К.15 Изменение потребляемой мощности в зависимости от перепада давления при средней скорости агрегата Vср = 1,8 м/с

Рисунок – К.16 Изменение потребляемой мощности в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 1,8 м/с

Рисунок – К.17 Изменение перепада давления в зависимости от оборотов гидромотора при средней скорости агрегата Vср = 1,58 м/с

Рисунок – К.18 Изменение потребляемой мощности в зависимости от поступательной скорости агрегата

Как видно из графиков, мощность на привод приводного механизма агрегата растет с увеличением перепада давления и увеличением оборотов гидромотора (рисунки К.6 – К.17). Зависимость изменения потребляемой мощности на привод агрегата, при поступательной ее скорости изменяется почти по линейному закону (рисунок К.18). Из графика видно, что с увеличением поступательной скорости агрегата потребляемая мощность на привод механизма агрегата падает с 1,86 кВт до 1,52 кВт, что на 18% меньше чем при средней скорости 0,52 м/с. При намотке мульчирующей пленки и гибких капельных лент, предохранительные муфты приводного механизма агрегата срабатывают в зависимости от поступательной скорости агрегата. Таким образом, с увеличением поступательной скорости агрегата, время срабатывания предохранительных муфт сокращается, что приводит к уменьшению потребляемой мощности на привод.

Подача освобождённой пленки (от почвы) и скошенной массы растений по наклонному транспортеру контролировалась по перемещению ленты вместе с наклонным транспортером. При этом отсутствовало обратное падение скошенной массы назад по транспортеру. Это показывало угол наклона транспортера подобрана в пределах нормы относительно горизонта.

Отделение скошенной массы растений и остатков земли от мульчирующей пленки, направленной для намотки на поверхности барабана, происходило под воздействием инерционной силы. Куски почвы, залипшие на поверхности при повороте мульчирующей пленки в концевом ролике транспортера, отлепились, также остатки растительных отходов отделились.

Намотка мульчирующей пленки на поверхность барабана происходила под натяжением независимо от незначительного изменения движения агрегата. Этому способствовало завышенные обороты барабана от гидропривода.  При этом от разрыва мульчирующей пленки сохраняла предохранительная муфта, отрегулированная заранее на необходимый крутящий момент барабана. Определялась начальная длина пленки до намотки на барабан с помощь линейки в начале каждого загона и данные заносились в ведомость (таблица К.2).

По полученным данным испытаний определялись суммарные значения замеров и их средние значения.

Таблица К.2 – Ведомость определения глубины выкапывания почвы у края пленки и начальная длина пленки до намотки

| Учетная делянка | Глубина выкапывания, см | Начальная длина до намотки, см |
| --- | --- | --- |
| 1 | 12 | 50 |
| 2 | 11 | 70 |
| 3 | 13 | 50 |
| Сумма | 36 | 170 |
| Среднее значение | 12 | 56,6 |

Измерение тягового сопротивления экспериментальной машины проводились методом динамометрирования посредствам буксировки пахотного агрегата вторым трактором (рисунок К.19).

Тяговое сопротивление машины для удаления мульчирующей пленки и гибкой капельной ленты замерялось при помощи динамометра для определения тягового усилия машин марки МЭЗИМ с предельной нагрузкой до 3000 кг.

Рисунок – К.19 Испытание тягового сопротивления агрегата

Тяговое сопротивление сельскохозяйственной машины Rм, вычислялось по формуле

| Rм = Rт.а. - Rт.с., Н | (А.2) |
| --- | --- |

где Rт.а. - тяговое сопротивление машинно-тракторного агрегата при выполнении технологической операции, H;

Rт.с. - тяговое сопротивление трактора при его движении без сельскохозяйственной машины, H.

Результаты тяговых испытаний после математической обработки представлены таблице К.3.

Таблица К.3 – Показатели тягового сопротивления агрегата

| Наименование измерения | Номер испытания |  |  |  |  | Среднее значение | Коэффициент вариации |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |  |  |
| Rт.а., кН | 6,35 | 6,39 | 6,36 | 6,37 | 6,38 | 6,37 | 0,002482 |
| Rт.с., кН | 4,42 | 4,41 | 4,39 | 4,43 | 4,4 | 4,41 | 0,003585 |
| Rм, кН | 1,93 | 1,98 | 1,97 | 1,94 | 1,98 | 1,96 | 0,023452 |

Сброс намотанной мульчирующей пленки и гибкой капельной ленты проводилось после прохождения агрегатом длины гона и заправлялась сердечником (барабаном) для последующего заезда (рисунок К.20).

Рисунок К.20 – Вид намотанной мульчирующей пленки и гибкой капельной ленты в момент сброса из агрегата

Испытание механизма для уборки гибких поливных лент капельного орошения проводились после отделения путем демонтажа от основного устройства и установлением на навесное устройство трактора.  Такой способ необходим обычно для случая, когда отсутствует мульчирующая пленка. Сбор ленты проводился в стационарном состоянии трактора у края поля. Навесной механизм испытывался с сердечником и без сердечника намотки ленты.  При этом использовался направляющее устройство для намотки ленты (рисунок К.21).

Рисунок К.21 - Намотка гибких поливных лент капельного орошения

По результатам экспериментальных исследований машины для удаления мульчирующей пленки и гибкой капельной ленты,  были определены геометрические и динамические параметры (таблица К.4).

Таблица К.4 – Краткая техническая характеристика машины для удаления мульчирующей пленки и гибкой капельной ленты.

| Ширина пленки, м | до 1,5 |
| --- | --- |
| Ширина захвата агрегата, м | 1,5 |
| Производительность, га/ч | до 1,2 |
| Рабочая скорость, м/с | до 1,8 |
| Частота вращения вала гидромотора, об/мин | 150…170 |
| Масса, кг | 550 |

APPENDIX L

Economic efficiency of work

Для экономической оценки можно было сравнивать три варианта удаления мульчирующей пленки и гибкой поливной ленты:

- базовая рассадопосадочная машина,

- удаление мульчирующей пленки и гибкой капельной ленты вручную,

- экспериментальный образец машины для удаления мульчирующей пленки и гибкой капельной ленты.

Для сравнительной оценки были выбраны согласно ГОСТ 34393-2018 только механизированные способы посадки рассады. За аналог для сравнения принимают лучшую серийно выпускаемую в государстве технику, а при его отсутствии – лучший зарубежный аналог, который преобладает в составе зональных машинных технологий в конкретном государстве.

В качестве аналогового варианта, использовалась показатели машины PMR-01 компании «Rocca Industries Pty Ltd».

Технические характеристики для расчета представлены в таблице Л.1.

Таблица Л.1 - Основные показатели машин для сравнительной оценки

| Показатели | Аналог | Новый |
| --- | --- | --- |
| Ширина захвата, м Производительность, га Масса, кг Количество обслуживающего персонала, включая тракториста, чел. Стоимость, тыс. тг | до 1,5 до 1,2 540 2 10 298,5 | до 1,5 до 1,2 550 1 1 700,0 |

Определение преимуществ и недостатков проектируемого варианта технологии, техники, ее обслуживания и ремонта заключается в сопоставлении по вариантам частных и общих экономических показателей, характеризующих экономию общественного труда.

Л.1 Основные показатели экономической оценки

На выполнение единицы вида работы экономические показатели включают в себя:

- затраты труда,

- прямые эксплуатационные затраты денежных средств,

- совокупные затраты денежных средств.

Показатели ресурсосбережения включают в себя:

- совокупные затраты денежных средств на годовой фактический объем работы новой техники;

- выполнение годового условного объема работы, в том числе:

1) совокупные затраты денежных средств;

2) потребность в технике;

3) потребность в обслуживающем персонале (механизаторах и вспомогательных рабочих);

4) потребность в дизельном топливе;

5) потребность в капитальных вложениях.

Показатели сравнительной экономической эффективности включают в себя:

- годовую экономию совокупных затрат денежных средств в сравнении с аналогом,

- снижение себестоимости работы,

- срок окупаемости капиталовложений,

- снижение потребности в обслуживающем персонале (механизаторах и вспомогательных рабочих),

- снижение потребности в дизельном топливе.

Л.2 Методы определения экономических показателей на отдельных видах механизированных работ

Экономические показатели по специализированной технике, комбинированным агрегатам и универсальной технике на отдельных видах (удаление мульчирующей пленки и гибких поливных лент) механизированных работ определялись в расчете на единицу наработки (га).

Затраты труда на вид работы , чел.-ч/ед. наработки, определялись по формуле:

,                                                             (Л.1)

где - общее количество механизаторов и вспомогательных рабочих, обслуживающие самоходную технику в течение смены, чел.;

- производительность МТА за 1 ч сменного времени га/час.

Прямые эксплуатационные денежные затраты, приходящиеся на выполнение одной единицы работы , тг/га., вычислялись по формуле

,                                     (Л.2)

где - затраты денежных средств для оплаты труда обслуживающего персонала, тг/га;

- затраты денежных средств для оплаты дизельного топлива тг/га;

- затраты денежных средств на техническое обслуживание и ремонт, тг/га;

- отчисления на амортизацию, тг/га;

- затраты денежных средств на вспомогательные необходимые технологические материалы, тг/га.

Денежные затраты для оплаты труда обслуживающего персонала вычислялись  по следующей формуле

,                                                            (Л.3)

где - обслуживающий персонал -й квалификации, чел.;

- оплата труда обслуживающего персонала за час  -й квалификации,  тг/чел.-ч;

- коэффициент, с учетом уровня социальных отчислений от полной зарплаты, (0,35 в РК);

- общее количество обслуживающего персонала, чел.

Затраты денежных средств на оплату горюче смазочных материалов определялись по формуле

,                                                         (Л.4)

где - удельный расход дизельного топлива, кг/га;

- цена за дизельное топливо, тг/кг;

- коэффициент учета цен на смазочные материалы.

Вычисление затрат денежных средств на техническое обслуживание и ремонт техники определялись по формуле

,                                                          (Л.5)

где - количество техники, входящих в МТА, шт.;

- цена машины для удаления мульчирующей пленки без учета НДС, тг;

- отчисления на техническое обслуживание и ремонт от цены машины для удаления мульчирующей пленки на 100 ч ее работы, % (7,37 в РК) [118];

- производительность машины для удаления мульчирующей пленки за 1 ч эксплуатационного времени, га.

Производительность машины для удаления мульчирующей пленки за 1 ч эксплуатационного времени вычислялись по формуле

,                                                    (Л.6)

где - производительность машины для удаления мульчирующей пленки за период контрольных смен за 1 ч основного времени, га.;

- коэффициент использования сменного времени машины для удаления мульчирующей пленки за период контролирующих смен;

- коэффициент готовности по оперативному времени машины для удаления мульчирующей пленки.

Отчисления на амортизацию вычислялись по формуле

,                                                                  (Л.7)

где - значение амортизационного ресурса для комбинированных агрегатов равно - 1260, ч.

Расходы денежных средств на необходимые технологические материалы  определялись по формуле

,                                                          (Л.8)

где - удельный расход вспомогательного технологического материала, кг/га;

- цена за одну единицу вспомогательного технологического материала, тг/кг;

- число наименовании вспомогательных материалов, шт.

Сумма денежных средств на совокупные затраты выполнения работ на гектар  , тг/га, вычислялись по формуле

,                                (Л.9)

где - расходы денежных средств от потери сельскохозяйственных продукции, тг/га;

- расходы денежных средств при повреждении сельскохозяйственных продукции, тг/га;

- расходы денежных средств от неправильного применения рассады, тг/га;

- расходы денежных средств на охрану окружающей среды, тг/га.

Расходы денежных средств от потери сельскохозяйственных продуктов, вычислялись по формуле

,                                                      (Л.10)

где - урожайность выращиваемой культуры, т/га;

- потери выращиваемой культуры при применении МТА, %;

-  цена полноценной сельскохозяйственной продукции на рынке, тг/т.

Расходы денежных средств при повреждении выращиваемой культуры вычислялись по формуле

,                                         (Л.11)

где - повреждение и дробление выращиваемой культуры, %;

- цена поврежденной и дробленой выращиваемой культуры на рынке, тг/т.

Расходы денежных средств от неправильного применения посадочных рассад вычислялись по формуле

,                                           (Л.12)

где - норма посадки рассад по факту, шт/га;

- количество рассады с продуктивным всходом, шт/га;

- цена рассады тг/шт.

Количество рассады с продуктивным всходам, можно было вычислить по формуле

,                                                         (Л.13)

где - относительная полевая всхожесть рассад, % (определялись по результатам лабораторно-полевых испытаний рассадопосадочной машины).

Затраты денежных средств на охрану окружающей среды на 1 гектар, вычислялись по формуле

,                                                            (Л.14)

где - норма отчислений на охрану окружающей среды за выброс двигателями внутреннего сгорания в атмосферу грязных веществ, тг/кг.

Л.3 Определение показателей ресурсосбережения

Совокупные затраты денежных средств при годовом фактическом объеме работы одного МТА, выполняющего на поле вид работы за сезон, , тг, вычислялись по формуле

,                                                     (Л.15)

где - годовой фактический объем работ МТА, га.

Годовой фактический объем работы в составе МТА, выполненный в сезон при одном МТА, с учетом агротехнических сроков в Республике Казахстан, вычислялись по формуле

,                                                      (Л.16)

где - агротехнический срок посадки всей рассады, дней;

- максимальное время работы МТА в сутки на поле, ч.

При выполнении годового условного объема посадки рассады, совокупные затраты денежных средств , тг, вычислялись по формуле

,                                                    (Л.17)

где - годовой условный объем посадки рассады (1000 гектар), га [117].

Необходимая потребность техники для выполнения годового условного объема работы , шт., вычислялись по формуле

.                                                                (Л.18)

Годовую потребность в механизаторах и вспомогательных рабочих при годовом условном объеме выполнения работ, чел., вычислялись по формуле

,                                                     (Л.19)

где - количество смен работы обслуживающего персонала за сутки, шт.

Годовая потребность в дизельного топливе на годовой условный объем работы , кг, вычислялись по формуле

.                                                                (Л.20)

Капитальные вложения  при выполнении работы  , тенге, вычислялись по формуле

,                                       (Л.21)

где - цена энергетического средства, тенге;

- цена машины для удаления мульчирующей пленки, тенге;

-  количество сельскохозяйственных машин в МТА, шт.

А4 Определение показателей сравнительной экономической эффективности

Годовая экономия совокупных затрат денежных средств при выполнении годового фактического объем работы МТА, при удалении мульчирующей пленки, , тенге, вычислялись по формуле

,                                         (Л.22)

где , - совокупные затраты денежных средств при выполнении годового фактического объема работы МТА при выполнении работы, для аналога и новой технике, тенге.

Годовая экономия совокупных денежных затрат при выполнении  годового условного объема  работы , тенге, вычислялась по формуле

,                                            (Л.23)

где , - совокупные денежные затраты при выполнении годового условного объема работ для  аналога машины  и предлагаемой новой техники, тенге.

Снижение себестоимости выполнения работы, %, определяли по формуле

.                                         (Л.24)

Срок окупаемости капитальных вложении техники входящих в МТА при выполнения годового условного объема работы , лет, определялись по формуле

,                                (Л.25)

где - производительность новой машины для удаления мульчирующей пленки при выполнении работы сменного времени, га/ч;

- цена нового энергетического средства, тенге;

- цена новой машины для удаления мульчирующей пленки, тенге;

- амортизационный ресурс нового энергетического средства, ч;

- амортизационный ресурс новой машины для удаления мульчирующей пленки, ч.

Снижение количества в механизаторах и вспомогательных рабочих при выполнении годового условного объема работы , %, вычислялись по формуле

,                                                        (Л.26)

где , - годовая необходимость в механизаторах и вспомогательных рабочих при выполнении годового условного объема работы для предлагаемой новой технике и аналоговой технике соответственно, чел.

Снижение потребности в дизельном топливе при выполнении годового условного объема работы , %, вычислялись по формуле

,                                                     (Л.27)

где , - годовая потребность в дизельном топливе на годовой условный объем работы для предлагаемой новой и аналоговой технике соответственно, кг.

ГОСТ 34393-2018.  В таблицах Л.2 – Л.4 приведены результаты расчета сравнительных показателей экономической эффективности при выполнении операции укладки мульчирующей пленки   предлагаемой новой техникой в сравнении с применяемым аналогом.

Таблица Л.2 - Показатели экономической оценки на 1 гектар

| Наименование показателя | Значение показателя по |  |
| --- | --- | --- |
|  | новой | аналогу |
| Вид механизированной работы | удаление мульчирующей пленки | удаление мульчирующей пленки |
| Марка техники (состав МТА) | МТЗ-80 Эксп. образец | МТЗ-80 Rocca PMR-01 |
| Производительность МТА при сменном времени, га/ч | до 1,2 | до 1,2 |
| Удельный расход дизельного топлива кг/га. | 15 | 15 |
| Совокупные затраты денежных средств всего, тенге/гектар, в том числе: - оплата труда - покупка дизельного топлива - техническое обслуживание - амортизационные отчисления - затраты на вспомогательные материалы - потери основной продукции томата - повреждения продукции томата - нерациональное использование посадочного материала (рассада) - охрана окружающей среды | 1 750 11 001 755,93 1124,34 0 1 308 336 430 214,4 18 000 19,50 | 3 500 11 001 4 579,40 6 811,18 0 1 308 336 430 214,4 18 000 19,50 |
| Совокупные затраты денежных средств на 1 гектар, тенге | 1 771 201,17 | 1 782 461,48 |

Таблица Л.3 - Показатели ресурсосбережения

| Наименование показателей | Значение показателя для |  |
| --- | --- | --- |
|  | новая | аналог |
| Совокупные затраты денежных средств на годовой фактический объем работы новой техники, тенге | 104 146 628 | 104 808 734 |
| Годовой фактический объем вида работы, га | 58,8 | 58,8 |
| Годовой условный объем работы, га | 1000 | 1000 |
| Совокупные затраты на годовой условный объем работы, тенге | 1 771 201 171 | 1 782 461 476 |
| Потребность в капиталовложениях на годовой условный объем работы, тенге | 53 500 000 | 96 492 500 |
| Потребность в обслуживающем персонале на годовой условный объем работы, человек | 7 | 14 |
| Потребность в дизельном топливе на годовой условный объем работы, кг | 15 000 | 15 000 |

Таблица Л.4 - Основные показатели сравнительной экономической эффективности на условный объем работы

| Наименование показателей | Показатели новой техники |
| --- | --- |
| Годовая экономия совокупных затрат денежных средств, тенге | 11 260 304,96 |
| Снижение себестоимости выполнения работы, % | 0,63 |
| Срок окупаемости капитальных вложений, лет | 1,24 |
| Снижение потребности в механизаторах и вспомогательных рабочих, % | 50 |
| Снижение потребности в дизельном топливе , % | 0 |

Таблица Л.5 – Данные сравнительных испытаний в программе Excel

Продолжение таблица М.5

Продолжение таблица М.5

APPENDIX M

Copy of articles

М.1 Оттиски публикаций за 2018 год

М.2 Оттиски публикаций за 2019 год

М.3 Оттиски публикаций за 2020 год

APPENDIX N

Equipment Certificates

APPENDIX P

List of articles published in 2018 -2020

1. Муратбекқызы Т., Хазимов К.М. Удаление мульчирующей пленки и гибких поливных лент капельного орошения с поля в послеуборочный период//Сборник международной начно-практической конференции молодых ученых, проведенная в рамках «Международной магистерской летней школы» – Алматы, 2018.- С. 174-180.

2. Хазимов Ж. М., Хазимов К.М., Хазимов М.Ж. Исследование влажности и температуры почвы поля при механизированной технологии посадки рассады овощных культур с использованием мульчирующей пленки в период выращивания//Вестник науки – Томск, 2018.- С. 38-44.

3. Муратбеккызы Т., Ахметканова Г.А., Хазимов К.М., Хазимов М.Ж.Устройство для снятие полимерных остатков в послеуборочный период овощей // Цифровизация агропромышленного комплекса, том II. – Тамбов: Издательский центр ФГБОУ ВО «ТГТУ», 2018 г. – С.51-53.

4. Z.M. Khazimov, G.C. Bora, K.M. Khazimov, M.Z. Khazimov, I.B. Ultanova, A.K. Niyazbayev Development of a dual action planting and mulching machine for vegetable seedlings// Engineering in Agriculture, Environment and Food. Volume 11, Issue 2, April 2018, P. 74-78. https://doi.org/10.1016/j.eaef.2018.02.003

5. Ниязбаев А.К., Ахметканова Г.А., Николаева К.Н. Механизация удаления мульчирующей пленки с поверхности почвы в послеуборочный период. // XXIII Международная научно-практическая конференция молодых ученных и студентов «Научная молодежь в аграрной науке: достижения и перспективы» Сборник материалов. – Алматы. – 2019. – С. 51-56.

6. Ниязбаев А.К., Хазимов К.М., Сафаргалиев А.Е., Базарбаева Т.А., Урымбаева А.А. Удаление мульчирующей пленки и гибкой поливной ленты механизированным способом с поверхности поля в послеуборочный период. // Исследования, результаты. – Алматы. – 2019. – №2. – С. 372-381.

7. M.Z. Khazimov, K.M. Khazimov, T.A. Bazarbayeva, A.A. Urymbayeva, G.C. Bora, A.K. Niyazbayev Mechanization of removal of the mulching film and flexible irrigation tape from the surface of the fields // Eurasian journal of bioscience. Volume 13. Issue 2. – 2019. – P. 1251-1261.

8. Ниязбаев А.К., Хазимов М.Ж., Хазимов К.М., Сафаргалиев А.Е., Урымбаева А.А. Обеспечение экологической безопасности в сельском хозяйстве // Вопросы географии и геоэкологии. – Алматы. – декабрь 2019. – №3. – С. 80-85.

9. Ниязбаев А.К., Хазимов М.Ж., Бора Г.Ч. Исследование сопротивления почвы пр намотке мульчирующей пленки // VІII Международная научно-практическая конференция «GLOBAL SCIENCE AND INNOVATIONS 2020: CENTRAL ASIA» Сборник материалов. – Нур-Султан. – 2020. – С. 113-117.

10. Ниязбаев А.К., Хазимов М.Ж., Хазимов К.М. Интенсификация производства овощей путем использования мульчирующей пленки и гибких поливных лент капельного орошения // Исследования, результаты. – Алматы. – 2020. – №3. – С. 395-402.

11. Хазимов М.Ж., Ниязбаев А.К., Хазимов К.М., Урымбаева А.А., Оханов Е.Л. Исследование изменений прочностных показаетелей полимерных пленок от атмосферных и биологических факторов // Исследования, результаты. – Алматы. – 2020. – №3. – С. 452-459.

12. K.M. Khazimov, M.Zh. Khazimov, A.K. Niyazbayev, G.A. Akhmetkanova, A.A. Urymbayeva, G.A. Mukanova, T.A. Bazarbayeva, V.F. Nekrashevich Design of a novel device for plastic mulch retriever // Journal of Water and Land Development. Volume 47. Issue 4. – 2020.

13. 6. R. Barzin, R. Pathak, H. Lotfi, J. Varco, G.C. Bora Use of UAS multispectral imagery at different physiological stages for yield prediction and input resource optimization in corn // Remote Sensing. Volume 12. Issue 15. 2020. https://doi.org/10.3390/rs12152392

14. Хазимов М.Ж., Хазимов К.М., Ниязбаев А.К. Технология и средства удаления мульчирующей пленки и гибких поливных лент капельного орошения с поверхности поля в послеуборочный период при интенсивной технологии производства овощей (монография). – Алматы: Алма Принт. 2020. – 159 с.